SOLID STATE

INDUCTION HEATING EQUIPMENT

OPERATION AND

MAINTENANCE INSTRUCTIONS

NovaStar

3kW Power Supply

| File Name | input.doc.doc |
| --- | --- |
| Part Number | 801-9061 |
| Revision | K |
| ECN# | 0208-19 |
| By | WmF |
| Approved |  |
| Date | 09/26/2002 |

AMERITHERM INC.

39 Main Street, POBox 901

Scottsville, NY  14546

tel 800.456.4328

fax 585.889.4030

www.ameritherm.com

Derivation:

This K revision has been written to:

- describe added inductor L3

Table of Contents

1	How to Unpack and Inspect	5

1.1	Power Supply	6

1.2	Heat Station	6

2	What Is Your NovaStar System?	7

2.1	Mechanical	9

2.2	Electrical	10

2.3	Environmental	11

2.4	Front Panel	12

2.5	Rear Panel	15

3	Inside Your Power Supply	19

3.1	Component Description	20

3.2	Circuit Boards	22

3.3	Cable Map	24

4	How to Install Your System	25

4.1	Site Requirements	25

4.2	Power Supply and Cabling	26

4.3	Heat Station Connections	28

4.4	Safety Considerations	31

5	How Your System Works	33

5.1	Sequence	33

5.2	Setting the Output Levels	34

5.3	When a Fault Occurs	37

6	Customizing Your System	39

6.1	Changing Setup Switches	39

6.2	Choosing Start and Reset Options	44

6.3	Connecting Your Pendant	46

6.4	Setting Output Control Options	50

6.5	Connecting Your PLC	52

7	Theory of Coil Design	55

8	Serial (RS-485) Control	57

9	Appendices	65

9.1	How to Replace Fuses	65

9.2	How to Order Replacement Circuit Boards	68

9.3	How to Clean the Air Filter	68

9.4	Heat Station maximum ratings	69

9.5	How to Use Matching Capacitors	70

9.6	How to Re-select Line Voltage	71

Notes:

In this manual, these symbols indicate:

|  | a step in a procedure |
| --- | --- |
| § | manual sections |
|  | lighted indicators on the equipment |

CAUTION: directing your attention to specific issues that impact the operation of the equipment. Failure to observe a CAUTION can result in diminished equipment efficiencies and/or failure.

WARNING: directing your attention to issues of electrical hazards impacting personal and plant safety. Failure to observe WARNINGs can result in fire, injury or death.

# How to Unpack and Inspect

Your Ameritherm NovaStar Induction Heating system has been carefully packed to arrive at your facility in good condition.

We suggest you inspect the shipping carton the moment the unit arrives at your loading dock and in the presence of the carrier.  Look for dents, crushed corners, or torn cartons.

YOU MUST REPORT ALL DAMAGE DIRECTLY TO THE CARRIER.

Check to see that all the parts we shipped arrived at your plant:

- NovaStar power supply

- Heat Station with RF power and sensing cables attached  (10 ft is standard; you may have ordered more)

- Operation and Maintenance Instructions (this Manual)

- Schematic Drawing set

- Installation Drawing set

- Test-Coil and Data Sheet (optional)

- Work-Coil and drawings (optional, per your specification)

- Pendant controller (optional, §6.3)

If you note damage, do not discard any of the exterior packing cartons; retain the carton(s) so that the carrier's claim agent may inspect it.  Ameritherm will assist you in filing a claim against the carrier should it be necessary to do so.

If no external damage is noted

 cut the banding material and remove the external carton.

 Do not cut the carton; this may scratch or mar the enclosure finish and/or the front panel instrumentation.

 lift the NovaStar power supply out of the carton

## Power Supply

Once the equipment has been removed from the carton:

 Inspect the front panel instrumentation for any signs of damage.

 Examine the rear panel for signs of shipping damage

 Unpack any additional equipment that may be packed and shipped in separate cartons. Inspect them and note any damage. If you are unsure whether components are missing, call our factory (in the US) at:

800.456.HEAT

or

716.889.9000

fax 716.889.4030

## Heat Station

 Carefully remove your Heat Station (packed separately) from the carton and inspect it for damage. If you note any damage, retain the shipping carton so that the carrier's claim agent may inspect it.

 Packed with the Heat Station, find and remove the following:

- Work-coil mounting kit (O-rings, etc.)

- Push-button pendant controller (optional)

REPORT ANY SHORTAGES TO AMERITHERM

# What Is Your NovaStar System?

It is a solid-state induction heating system complete in two components:

- Power Supply

- Heat Station

The air-cooled or water-cooled power supply converts AC line voltage to a 3kW output over a range of radio frequencies and voltages. This energy is delivered over the RF power cable to the water-cooled Heat Station where it creates a magnetic field around your work-piece, heating it with precise control of temperature and timing. (See Notes:, page 4)

- When first energized (Fault), a manual check of safety circuits is required.

- RESET makes this check and returns Ready if OK.

- Heat On is then initiated with a START signal

- RF energy is delivered to the Heat Station during this heat cycle.

- STOP or E-Stop signals halt the RF energy.

- Non-routine operation is sensed and annunciated with a Fault and is further described by a CODE, displayed at the front panel.

NovaStar uses a proprietary tank sensing technology which enables you to exercise greater control of your process; precise frequency registration under varying load conditions results in improvements in the precision in heat control and times-to-temp.

Your power supply can be operated in any of the following ways:

- Manually from the front panel

- Remotely from a pendant controller (optional)

- Remotely from your PLC

- Remotely from your RS-485 device

An RS-485 full-duplex port (four-wire) - requiring only a few short commands - provides flexibility in remote operation and monitoring of your induction process.

Your power supply has been assembled with one or more of the following options:

- 220 or 440 V line operation

- Air-cooling or water-cooling

- Rack-mounting or bench-mounting

- Remote front panel kit

- Temperature controller

FUNCTIONAL DIAGRAM

## Mechanical

Power Supply

| Feature | Value |  |  | Units |
| --- | --- | --- | --- | --- |
| Dimensions HxWxD | 9 x 17 x 27 (22.9 x 43.2 x 68.6) |  |  | in (cm) |
| Internal access | 10 counter-sunk ¼-turn Philips-head screws |  |  |  |
| Mounting | Rack-mount or bench-top options |  |  |  |
| Weight | 220V model |  | 440V model |  |
|  | 87 (39.5) |  | 97 (44) | lb. (kg) |
| Construction: | chassis cover panels | 14 ga CR steel with welded seams 16 ga CR steel 0.080 or 0.125” aluminum |  |  |
| Finish | exterior interior: | hybrid powder-coat zinc under-finish |  |  |

Your NovaStar system uses and produces high voltages! Only trained or guided service persons are authorized inside the Supply. Portions inside the Supply remain energized and hazardous after the AC Power Switch is off. Always turn off the customer-supplied fused disconnect before attempting any service.

Heat Station

| Feature | Value | Units |
| --- | --- | --- |
| Dimensions HxWxD | 4.5 x 4 x 8 (114 x 102 x 201) typ | in (cm) |
| Weight | 12 (5.4) typ | lb. (kg) |
| Final specifications may vary with application.. |  |  |

RF Power & RF Sensing Cables

|  | Standard | Optional |  |
| --- | --- | --- | --- |
| Length | 10 (3) | 25 – 200 (7.6 – 61) | ft (m) |
| Weight | 5 (2.3) |  | lb. (kg) |

The RF power cable is constructed to specific requirements; it is a specialized cable. Do not substitute any other cable in its place. Failure to observe this caution can result in severely diminished performance and in cable over-heating. Call Ameritherm to replace or re-specify the RF power cable.

## Electrical

Equipment Input

| Description | Value |  |  |  |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
| AC line voltage, 3 1 | 220 | 380 | 400 | 440 | 480 | Vrms, 10% |
| Current, full-load | 13 | 7.5 | 7.0 | 6.5 | 6 | Arms/ |
| Internal OCP (3 fuses) | 15 | 10 |  |  |  | Arms |
| Frequency | 50 - 60 |  |  |  |  | Hz |
| Real Power (Pin) | 3.8 |  |  |  |  | kW |
| Apparent Power | 4.5 |  |  |  |  | kVA |
| Power Factor | .87 |  |  |  |  | typical |

1.) Line selection is factory-set per customer order. To re-select voltage, internal transformer must be tapped per §9.

Customer Input

| Port | Enabled | Active | Description | Limits |  |
| --- | --- | --- | --- | --- | --- |
| START |  | HI | Optical isolated 2500Vrms, used to control by external dc voltage | max 24Vdc, min ON: 16.8Vdc@7mA max OFF: 1V |  |
| STOP |  | LO |  |  |  |
| INP1, INP2 | optional, then  | * |  |  |  |
| RESET |  | N.O. | pend. or cust. switch, mom. closed contact = RESET | 24Vdc open circuit 5 mA short circuit max |  |
| HS Door |  | N.O. | limit switch @ HeatStation, non-isolated loop signal, opened contact = Fault | 24Vdc open circuit 100mA short circuit max |  |
| INPUT SIGNAL |  |  | customer’s analog source, switches inform source type | 4-20mA into 250 0-20mA into 250 0-5Vdc into 100k 0-10Vdc into100k |  |
| FLS1 – FLS5 |  | N.O. | water flowswitch(es) non-isolated safety signal, opened contact = Fault | 24Vdc open circuit, 5mA short-circuit max (each) |  |
| Remote E-Stop |  |  | pendant-mounted switch opened contact = Fault | 24Vdc open circuit 100mA short circuit max |  |
| Cust. E-Stop |  | N.O. | customer’s equipment opened contact = Fault | 24Vdc open circuit 100mA short circuit max |  |
| RS-485 |  |  | d-submin 9-pin female (half-duplex) COM port | 2 differential pairs |  |
|  = by RS-485 |  |  = always |  |  | = by switch |
| * = per customer need |  | N.O. = normally open |  |  |  |

Equipment Output

| Description | Value |  | Units |
| --- | --- | --- | --- |
| NovaStar3 model | 220V line | 440V line |  |
| Voltage, internal | 200 |  | Vdc max |
| Current, internal | 18 |  | Adc, max |
| Frequency | 50 - 1000 |  | kHz |
| RF Voltage | 4251 |  | Vrms max |
| Pload, deliverable | 3.0* | 3.0 | kW |
| Efficiency, line-load | 80 |  | %, typ |

RF voltage maximum figures may stress output equipment (cable, heat station) beyond their operating limits. Do not exceed output settings described in your application data from Ameritherm. Follow instructions (§5.2, §9.3) in this manual

* derate by –3W/kHz for f>710kHz

1.) refer to §5 and §9 and your Installation Drawing package for complete transformer seetings information

Customer Output

| Port | Enabled | Active... | Description |  | Limits |
| --- | --- | --- | --- | --- | --- |
| Ready |  | Closed | isolated transistor output, 0.3A available |  | 24Vdc fused @1A |
| Heat On |  | Closed |  |  |  |
| Fault |  | Closed |  |  |  |
| OUT1, OUT2 | optional, then  | * |  |  |  |
| 24V |  |  | output grounded |  | 24V2%, @1A |
|  = by RS-485 |  |  = always |  | * = per Customer need |  |

## Environmental

| Value | Range | Units |
| --- | --- | --- |
| Ambient (air-cooled) (water-cooled) | 32 to 95 (0 to 35) 32 to 131 (0 to 55) | F (C) |
| Air cooling | 70 @ 32 to 95 (35 @ 0 to 35) | f3/m @ F (l/s @ C) |
| Water |  |  |
| flow (Power Supply) | 0.75 (2.84) | g/m (l/m) |
| flow (Heat Station) | .375 (1.42) |  |
| pressure2 | 40 - 60 (2.76 – 4.13) | lb/in2 (bar) |
| temperature3 | 50 - 95 (10 - 35) | F (C) |
| pH | 7.0 - 9.0 | pH |
| conductivity resistivity @25C | less than 500 greater than 2000 | S/cm cm |

2.) You must assure that the differential (inlet minus outlet) pressure falls within this range.

3.) Water temperature must not fall below the dew point in any case; condensation may result in nuisance problems or damage to the equipment

## Front Panel

Your NovaStar power supply is fully controllable from the front panel. While either your optional PLC or RS-485 serial controller can be used to operate the power supply, full manual operation is standard as follows:

Emergency Stop

If you (or an operator or bystander) detect an emergency condition at the power supply or at the Heat Station, use the E-Stop to immediately suspend RF power generation and to remove power from the Heat Station.  The E-Stop remains tripped until you clear it by turning the red dial counter-clockwise until it pops out. After correcting any problems and clearing the E-Stop, you must press RESET, enabling the power supply to re-check all its safety circuits.

AC Power switch

This knob is shown in the O, or OFF position: AC power removed from nearly all the supply.  Your incoming 3 connections (at the rear of the supply) and the wiring between the EMI filter and the disconnect line-side (inside the supply) remain live until you open the customer-supplied fused disconnect.

When OFF, the supply can be locked-out and tagged for service; three holes are provided around the perimeter of the knob to accept locks or lock-out tags when the switch is OFF, preventing it from being turned to the ON position.

Selector, Display and Adjust Dial

NovaStar displays its operating figures and settings from the front panel using four multi-segmented LEDs and a SELECTOR switch.  The Adjust dial is used in combination with the SELECTOR settings to set the particular operating value(s).  It is speed-sensitive when adjusting the Timer: while pressing, turn faster to dial-in more numbers per turn.

| Selector | Display | Press & turn Adjust |
| --- | --- | --- |
| Heat Cycles | Number of completed heat cycles since the power supply was turned on. | This has no effect |
| Timer | Number of seconds remaining in a heat-cycle | Set timer: turn CW from 0.000 to activate the timer. To de-activate, press/turn CCW to 0.010 or less 9999 seconds maximum |
| Volts | Voltage (% scale Vdc) developed internally | Dial-in the desired voltage when controlled (§6.1). Turn CW to increase this setting (§2.2). |
|  | NOTE: Switches #6 and #7 determine if output VOLTS is to be regulated (§6.1). You can always display output VOLTS. |  |
| Frequency | Resonant frequency (kHz) at the tank while tuning and Heat On | Has no effect under manual operation |
| Status | The current STATUS code (see §5.3). If a code has been stored, you see Fault and the display alternating: NOW  CODE If no code has been stored, see OK | Reveal the preceding code. Press to display WAS  CODE |

Indicators

The NovaStar power supply can be in one of three states, indicated by a light (LED). These lights indicate machine state no matter how the supply is being controlled! When lighted, they indicate:

| Ready | The safety circuitry (see §4.4) checks OK and the supply is prepared to apply power to the Heat Station; the power supply is standing-by. |
| --- | --- |
| Heat On | The START command has been received and the supply has found resonance at the Heat Station. Supply is operating at the output level selected (§6.1). All safety circuitry checks OK. |
| Fault | The processor has sensed that something is wrong; power to the Heat Station has been interrupted and a CODE has been recorded for display under STATUS. If the fault occurred in the interlock chain, RESET is required (after the fault has been corrected) before the supply will return to Ready. The power supply can automatically return to Ready following correction of other faults. See §5.2 for other special Fault recovery instructions |

When heat cycles are rapid (faster than about 1/sec), there may be a delay in reporting by these front panel indicators.  Rear panel outputs (§2.5) do not experience this delay.

Controls

Your power supply can be controlled from the front panel with three buttons:

| START | Acts only while Ready, results in Heat On; instructs supply to deliver power to the Heat Station. Rear Panel Switches #2 and #3 determine whether START originates from the front, rear or RS-485 sources. (§6.1) |
| --- | --- |
| STOP | Acts only while Heat On, results in Ready; halts delivery of power to the Heat Station. STOP is always effective from the front panel and from the rear panel. Jumpers are installed (standard) at CTB1 if there is no remote STOP signal being used. |
| RESET | Acts only during Fault (interlock & power-up), moves to Ready if safety circuits OK. Rear Panel Switch #11 is used to determine the origin of the signal (see §6.1). |

## Rear Panel

(Shown with the AC wiring cover removed) All connections to and from the NovaStar are made at the rear of the power supply:

Line Connection

Your power supply requires 3 AC power (see §4.3) at 220 or 440 volts (specified prior to sale). With the AC wiring cover removed, your 1.25” (32mm) conduit is attached to the Line port provided and your 4-conductor cable is routed to the Line connections. Power and grounding connections are made in this chamber.

RF Connection

The RF energy created by the supply is routed to the Heat Station via the RF cable, which attaches here (see §4.2). With the AC wiring cover removed, your 1.25” (32mm) conduit is attached to the RF port provided and the RF cable is routed back to the power supply from the Heat Station.

Sense Connection

The supply regulates the output frequency by directly monitoring voltage and current at the Heat Station. A two-pair, shielded cable brings this ‘sense’ signal back along-side the RF power cable, emerging from the Sense port (see §4.3). From there, four connections to CTB3 are made. CTB3 is removable after connections have been made so servicing or moving your equipment is convenient.

Setup Switches

Two banks of eight switches enable you to custom-configure your power supply to operate in many different ways (see §6.1). Once you’ve determined your requirements, set the switches and forget them! The directions on the label guide you in setting up:

- START method and source

- input source and signal

- output regulation parameter

- RESET source

Input/Output Panel

All connections are made to CTB1 and CTB2 for:

- Serial port; your RS-485 hardware connection, full-duplex (see §6)

- Connections and fuses to your output devices (see §6.5)

- Connections from a pendant or PLC controller (optional, see §6.3).

- Signal connections from your analog controller (see §6.4).

- Connections from external flow-switches (see §4.3).

- Connections from pendant-mounted E-Stop (see §6.3).

- Connections from your auxiliary E-Stop (see §6.3).

- Shield connection from pendant cabling.

CTB1 and CTB2 are removable after connections have been made, so servicing or moving your equipment is convenient.

Auxiliary Ground

Some of your I/O connections may require a local grounding point to aid in noise-suppression. Shielded cables are used in these cases. This point is provided for you to make a grounding connection if you desire.

Auxiliary ground is NOT INTENDED for equipment safety ground connections!  Auxiliary ground is a convenience, or signal grounding point. Safety ground is provided within the AC wiring chamber only!  (§4.2)

Your NovaStar system uses and produces high voltages! Only trained or guided service persons are authorized inside the Supply. Portions inside the Supply remain energized and hazardous after the AC Power Switch is off. Always turn off the customer-supplied fused disconnect before attempting any service.

Caution: Stored electrical charge can persist at internal terminals for several seconds following disconnection of AC power! Allow 60 seconds after disconnection before touching or servicing these terminals.

Inside Your Power Supply

If you need to get into the power supply,

 Turn off the AC power switch

 Turn off the customer-supplied fused disconnect; after this step, all power is removed.

 Use a philips screwdriver to release ten (10) ¼-turn screws and lift off the cover.

 Check the location of these parts:

## Component Description

(Circuits are described in §3.2)

Line Connections

AC power (lines A, B, C) and safety ground connect to these terminals

RF Connections

The RF power cable attaches to these two terminal posts (above the line connections).

Discharge Assembly

We recommend this component for heating applications where:

- heat cycles are conducted at repetition rate over 1 Hz

- heating occurs at pulses <500 ms

This component is required for applications where:

- heat ramp-down is closely controlled (35 ms)

Residual charge is shunted through this resistor following each heat cycle. A thermal switch monitors resistor heating. Firmware control of the discharge circuitry is factory-enabled.

PS Interlock

A normally-open limit-switch responds when the cover has been removed. RF voltage synthesis is inhibited and much of the line voltage distribution is interrupted when the cover is open (see §4.3, Safety).

Inductor L3

High frequency signals generated by the RF Power Module are filtered by L3 from the Voltage regulator

EMI Filter

This filter is implemented to prevent electrical noise from being transmitted between your power lines and the equipment. It is essential that this filter is the first component of the system to connect to the power lines; therefore, a small portion of the internal wiring (through the filter to the line-side of DISC1, the AC power switch) is live when the power switch is off. Internal labeling repeats this warning.

RF Transformer

The high frequency voltage synthesized by the system is isolated and scaled with a specially designed transformer. The input to the transformer is from the RF power module. The output of this transformer connects to the RF connections, above.

Line Fuses

Each of the power phases is fuse-protected to open the circuit in the event of a severe over-current condition or short-circuit to ground (chassis). These are replaceable ‘midget’ fuses. See §9.1 to change fuses.

Control Fuses & Transformer

Two of the three incoming phases are tapped to provide control voltages for the system. These fuses protect this branch. If your power supply has been ordered for 220Vrms operation, there are two control fuses. 440Vrms supplied units have three control fuses mounted atop a step-down transformer.

24V Power Supply

Provides regulated (2%) voltage for control, safety and logic.  Output is chassis-referenced and is fused at 1A.

CON1

This switch directs line voltage through the full-wave rectifier to the voltage regulator.  CON1 is open when the system is first turned on.  RESET checks safety circuits and closes CON2 at the Soft Start PCB if conditions permit.  After a time-delay, CON1 is closed and CON2 is opened by the Soft Start circuit If a safety fault arises, CON1 is immediately opened.

DISC1

This is the AC power switch, located between the EMI filter and the line fuses. Remember that, when this switch is open, AC lines from here and upstream are live and hazardous unless the customer-supplied fused disconnect is also open.

## Circuit Boards

Your NovaStar system is comprised of 6 printed circuit board groups. There are no operator-serviceable parts inside the power supply and access to the insides must be restricted to trained or guided personnel. These PCB descriptions are provided to illustrate the operation of your equipment, not to encourage unauthorized exploration.

Main Electronics Board

This is the ‘nerve center’ of the power supply; all functions are controlled by or report to the MEB. Resonance sensing and tuning, acquisition and display of operating data, fault monitoring and analysis are some of the functions performed by the MEB

Voltage Regulator, Capacitor Filter

Three-phase line voltage is full-wave rectified, passed through the Filter PCB and directed to this circuit which conditions and adjusts the DC level, depending on the control signal from the MEB. The DC voltage produced here is fed to the RF power Module. High frequency feed-back is blocked by L3. Certain conditions of over-voltage and over-current are monitored and controlled by the Regulator.

Rear-panel

Input and Output signal isolation, fusing and conditioning take place on these circuits. Portions of the safety relay configuration are located on these PCBs. All I/O connections to the supply are made to the Rear-panel. RS-485 signals are connected at the rear of the unit.

RF Power Module

The Voltage Regulator PCB feeds its DC output to the RF power module where it is switched to produce an alternating voltage at RF frequencies. The MEB directs the switching of this voltage.

Front Panel & Display

The control data that you enter at the front panel and the operating information displayed there are passed to and from the MEB through these PCBs. E-Stop, controls and indicators are mounted on and their signals are routed through the front panel.

Soft Start/Safety

Whenever you RESET your power supply, the line voltage is routed to the regulator, initially through CON2, located on the soft start PCB. This PCB provides for the gradual charging of the input stage of the regulator. It then closes CON1 after a timed-delay, re-routing the line voltage directly to the regulator. This PCB also monitors  L1 for missing phase conditions.

## Cable Map

A network of cables interconnects the components and circuits inside your NovaStar power supply. This ‘map’ indicates where each cable is installed.

Under circumstances of normal use, there is no need to move or remove any of these cables, yet conditions of service may sometimes require getting involved at this level.

# How to Install Your System

You may elect to use Ameritherm’s Service Department to install your NovaStar equipment.  Our field service engineers can be scheduled — in advance — to arrive at your facility to assure that your equipment is correctly and efficiently installed.  (Service contracts are available which extend your warranty from one to two years.)  Prior to a scheduled installation, we ask that you put your system in place and connect your factory power and water facilities. Please call us at call our factory (in the US) at:

800.456.HEAT

or

716.889.9000

fax 716.889.4030

## Site Requirements

Take these factors into consideration as you determine where to install your system:

- Availability of AC power

- Proximity to the work area (RF power cable length)

- Availability of cooling water and drain

- Sufficient service area around the equipment

 Allow at least 3” (8cm) clearance for ventilation around air-cooled power supplies.

## Power Supply and Cabling

Electrical installations must be made by a qualified electrician and must conform to applicable safety codes.

(Refer to your Installation and Schematic Drawings)

Your NovaStar system accepts 3 line-voltage at 220 or 440 Vrms (specified on your Sales Order and declared on the equipment data tag). Internal primary and secondary fusing anticipates this supply voltage and the equipment nameplate specifies the primary fusing used in your system. Line voltage selection is factory-set per customer order. To re-select voltage, internal transformer must be tapped per §9.

 Plan a 4-wire routing for phases A, B, C and earth ground

The equipment requires a connection to earth ground and does not make use of neutral. Failure to connect earth (safety) ground will result in a potentially hazardous condition. Do not omit this step!

Disconnect

We recommend that you install a fused disconnect switch (not supplied by Ameritherm) in the branch circuit supplying the equipment. Base your selection of fuses on the equipment nameplate data.

 Determine correct fusing and wire gauge for your equipment

 Place the disconnect near the power supply and in sight of the operator

Conduit

We recommend that the AC power and RF wiring to your power supply and Heat Station be run through solid metallic conduit (EMT). To ensure the highest levels of electromagnetic immunity and to effectively limit electromagnetic emissions, solid conduit is required. For AC power and RF cabling, 1.25” (32mm) conduit cutouts are provided at the rear panel.

 If you elect to use a smaller conduit size for your AC wiring, use a reducer to accomplish this conversion at the rear panel

AC Power

 Use a small Philips screwdriver to remove 5 screws securing the wiring cover; remove and set the cover and screws aside

 Attach and secure the conduit and bushings (between the fused disconnect and power supply) required for your installation

 Route the wiring from your fused-disconnect to the Line Port of the power supply. Base your selection of wire gauge on the equipment nameplate data.

 Attach #10 ring-lugs to each of the four wires (labeled A, B, C & GND)

 At each of the four terminals, remove the nut-washer combination

 Apply the corresponding wire to the power terminals. There is no polarity to these three connections.

 Re-attach and tighten the nut-washer combination.

 Do not replace the wiring cover; move to the next step.

## Heat Station Connections

(Refer to your Installation and Schematic Drawings)

The RF power cable is a specialized cable. Do not substitute any other cable in its place. Failure to observe this caution can result in severely diminished performance and in cable over-heating. Call Ameritherm to replace or re-specify the RF power cable.

Your Heat Station is shipped (typically) with the RF power and sensing cables attached. These cables are already connected inside the Heat Station; you must complete the connection to the power supply

RF Cabling

 Attach and secure the conduit and bushings (between the Heat Station and power supply) required for your installation. We recommend the use of 1.25” (32mm) EMT for cooling and to accommodate both cables.

 Route the RF cables through the conduit from the Heat Station to the RF port at the rear of the power supply.

 At the rear of the power supply, remove the 2 RF nut-washer combinations. Do not remove the insulating card that is installed on the terminals

 Attach the wires to the terminals; there is no polarity to this connection.

 Re-attach and tighten the nut-washer combinations.

 Carefully feed the sense cabling through the grommet to CTB3.

 Observe the sense cable labeling and use a small screwdriver to complete the connections to CTB3.  The labeling assures that this polarized cable is correctly connected. This terminal strip can be removed from its place on the rear panel for convenience in servicing and moving the power supply.

 Replace the wiring cover; tighten the 5 screws used to secure the cover.

For certain applications we provide an optional Cap Matching kit. When the resonant frequency at the Heat Station is in the higher ranges, the intrinsic impedance of the output components can have an undesirable effect on performance.  Nulling capacitors attached to this circuit tune-out this impedance. This circuit attaches over the three RF output terminals shown here and cable connection is made with this circuit in place.  The capacitor arrangement and user instructions depend on your application.

Power Supply Cooling Water

If your power supply uses circulating water to cool the internal components, you must provide this service now. Units requiring this cooling water are equipped with two 3/8” compression fittings: bottom-right for water input and in the flow switch for water outlet.

 Observe the restrictions in the section above.

 Establish a cooling water source capable of 40psi pressure and at least 0.75 g/m (2.8 l/m) flow.

 Install a shut-off valve upstream and convenient to the power supply

 Complete the plumbing to the power supply (Inlet is the lower fitting, outlet is from the flow switch).

Heat Station Cooling Water

In the process of heating your work-piece, the current flowing in the Heat Station tank circuit heats the components of the tank circuit, too. It is necessary to dispose of this excess heat to prevent damage to the capacitor and coil which comprise the tank circuit.

Each Heat Station provides a means of directing fresh water into and through the components, enabling the water to pick-up the excess heat and transfer it, either to drain or to cooling to be recycled back to the equipment.

DO NOT USE DE-IONIZING WATER SYSTEMS, ANTI-RUST OR ANTI-FREEZE AGENTS!

Install a pressure regulator and a particulate matter filter (both non-ferrous) in the supply of your cooling water.

Follow your Installation Drawing to complete the cooling water connections. Use copper, rubber hose, stainless steel or PVC tubing. Do not use ferrous pipe for this system; it can lead to serious equipment damage!

 Follow these important restrictions:

 Establish a cooling water source capable of 40psi differential pressure across the system and at least 0.375 g/m (1.4 l/m) flow.

 Install a shut-off valve upstream and convenient to the Heat Station

 Complete the plumbing to the Heat Station.

Flow Monitoring

The amount of water flowing through your Heat Station must be monitored, since inadequate flow or loss of cooling water can result in damage to the equipment. (One flow-switch is always a part of the Heat Station, providing a two-wire (N.O.) signal to the power supply. Another switch may be provided for some applications.) When flow is adequate, the switch is closed, when flow falls off, the switch opens and the power supply senses it, halts RF power and indicates Fault (FLS2, §5.2).

 Route the two flow-switch wires to the power supply. It is not necessary (or recommended) to include them inside the RF conduit.

 Route the wires to the FLS2 terminals and trim.

 Strip the ends to 1/4” (6mm).

 Using a small screwdriver, secure the two wires to CTB2, as shown. There is no polarity to this connection

Your power supply has been factory-programmed to monitor the Heat Station flow-switch at FLS2. If you omit this connection, or if your cooling flow is inadequate, your equipment will fail to RESET (see Fault, above).

If your power supply is water-cooled, a flow switch is mounted on the rear to assure adequate cooling water flow to the internal components of the power supply. The input FLS1 is reserved for use in this case and the firmware is factory programmed to enable FLS1. The wiring from this flow switch is factory-installed. If your Heat Station is equipped with a ‘bypass block’, requiring an additional parallel water path, a flow-switch is required in that path.

## Safety Considerations

Depending on your model, your Heat Station uses RF energy in ranges from 50 to 2000 kHz to raise the temperature of your work-piece. Most of this RF energy is transmitted into the work-piece and some is transmitted into the air.

Physical Risks

During the Heat On cycle, high AC voltages are present at your work-coil. There are several risks, which we instruct you to protect against:

High Temperatures: take steps to prevent personal contact with the work-piece you are heating. Severe burns can result from contact at these temperatures!

High RF Voltages: we recommend that you provide protection against personal contact with the work-coil when it is energized or in the Ready state.

Induction burns: the energized work-coil causes nearby metals to become heated; DO NOT WEAR JEWELRY OR CARRY KEYS within a few inches or centimeters of the energized work-coil.

Interlocked Barrier

We recommend construction of an interlocked mechanical barrier around the Heat Station in order to minimize all of these risks. Use a normally-open switch which closes only when the barrier is fully closed. A Faraday cage can double as a mechanical barrier!

 Direct the wiring from your interlock switch (COM - N.O.) to CTB1,

 Remove the jumper that has been installed at HS DOOR.

 Connect the switch wiring to these points. There is no polarity.

With this wiring in place, the power supply will not become Ready until the barrier is closed and, once heating, will shut down immediately if the barrier is opened. This input is always enabled.

Faraday Cage

If, when the equipment is heating the work-piece, you experience unacceptable interference with surrounding equipment (AM radios, Hall-effect switches, etc.) you can reduce this interference by constructing a ‘Faraday cage’ around your Heat Station.

This is nothing more than a metal box that completely surrounds the work-coil. It can be made of copper or aluminum and may be perforated (.125 to .250” or 3 to 6 mm). The RF energy that would otherwise go into the air is conducted over the surface of the cage. Providing a ground connection for the cage is not necessary but may provide an additional measure of RF suppression.

At the frequencies your equipment operates, adequate suppression can result if the cage is constructed of perforated UNPAINTED Aluminum, 14 to 16 gauge. Follow these construction tips:

- Do not use steel for any portion; it can be heated by your work-coil.

- Allow 2” or 5cm  (or 2 times the coil diameter) clearance between any part of the work-coil and the cage.

- Make sure there are no open seams or cracks

- Make good connections where the conduit enters the cage.

- Avoid the use of PAINT or anodizing where panel surfaces meet; these are insulators and they defeat the cage’s purpose.

Safety Circuits

An electrical interlock chain is used to prevent Ready (or to shut down the output power) in the event of cooling water loss, opening of the chassis or external limit-switch, E-Stop trip or internal over-heating. Each component of this chain is – in essence – some form of switch, held closed during safe operation. Whenever any of these switches opens, the condition is sensed immediately and the output power is stopped.

This ‘loop’ is always enabled, requiring 5mA current from the internal 24Vdc source. Three available connecting points (HS Door, Cust. E-Stop, Pend. E-Stop) are factory-jumpered to complete the circuit. You remove these jumpers as needed to series-connect your optional safety equipment (note HS switch, below).

When your power supply is turned on, line voltage is withheld and Fault is shown. RESET must be pressed to enable the signal at CON1 and to pass the N.C. contact if CON1 is open. From there, the signal enables the Safety Relay circuit which, in turn, checks that all safety devices are closed. If so, a control signal closes CON2 located on the soft-start circuit, gradually charging the components on the voltage regulator circuit. After a time delay, CON1 is closed, routing line voltage to the rest of the system, resulting in the Ready state.

# How Your System Works

This section describes what you have to do to make your NovaStar system operate while in its basic configuration: manual control from the front panel.

Setup of the standard configuration is described in §6.1.

## Sequence

To begin, the system is shut down and cold:

- your fused-disconnect is open (OFF)

- the power supply is OFF

- cooling water at the Heat Station is OFF

| Do this... | Observe... |
| --- | --- |
|  Open water-valve for the system | check drain; assure water is flowing |
|  At the Heat Station, make sure there is no work-piece yet |  |
|  Check condition of (any) safety barriers or limit switches | All safety devices are clear and electrically closed |
|  Turn on the fused disconnect | none |
|  SELECTOR to STATUS | none |
|  Turn on the AC power switch, clockwise | The display shows: V1.03, for example  Fault NOW  PSPH  PSFS, repeats (this is normal) |
|  Select VOLTS |  |
|  Press and turn Adjust | @Display, dial-in voltage level |
|  Select TIMER | none |
|  Press and turn Adjust | @Display, dial-in 2.500 sec (or 0.000 if no timer is required) |
|  Select FREQUENCY |  |
|  Press RESET | wait a few seconds while system checks safety circuit, then...  Ready |
|  Press START |  Heat On |

## Setting the Output Levels

Your NovaStar uses a unique transformer to isolate and scale the RF output to your specific needs. A combination step-up, step-down geometry makes a wide selection of output voltages available to you through a simple adjustment procedure (see your Installation Drawing package, P\N 805-series).

Generally, Ameritherm has considered your application (coils and loading characteristics) and we have pre-set the RF transformer to its optimum setting for those conditions; there may never be any need for you to adjust the transformer taps. But if you change work-coils at the heat station or if you make your own work-coils it may become necessary to change the RF transformer taps.

Purpose of this procedure

The most efficient application of energy to your work-piece takes place when

VOLTS (%) = AMPS (%)

Never allow AMPS (%) to exceed VOLTS (%).

Do not exceed maximum heat station voltage (§9.4)

The power supply allows you to adjust the output voltage from the front panel and the RF Transformer to achieve this optimum relationship; excellent results are possible if AMPS (%) is slightly less than VOLTS (%). Remember these rules during this setup:

In some cases, this relationship is impossible to achieve (e.g., large coil around a small part). You are the best and final judge of the output relationship: if your parts are being heated to your satisfaction, don’t change the settings.

Getting Started

Your NovaStar system uses and produces high voltages! Only trained or guided service persons are authorized inside the Supply. Portions inside the Supply remain energized and hazardous after the AC Power Switch is off. Always turn off the customer-supplied fused disconnect before attempting any service.

Caution: Stored electrical charge can persist at internal terminals for several seconds following disconnection of AC power! Allow 60 seconds after disconnection before touching or servicing these terminals.

 Turn off the AC power Switch

 Turn off the customer-supplied fused disconnect; after this step, all power is removed.

 Turn off all water supplied to the unit.

 Use a philips screwdriver to release ten (10) ¼-turn screws and lift off the cover.

 Assure that your work-coil is installed at the heat station. Remove work-pieces.

 Assure the following wiring at the RF Transformer; this is a starting point.

| [62] | [63] | [64] | [65] | [66] | [67] |
| --- | --- | --- | --- | --- | --- |
| A0 | A4 | L5 | B4 | B0 | L2 |

 Replace, close and secure the cover, restore water and AC power,

Inductor Settings

Refer to your Installation Drawing

| Do this... | Observe... |
| --- | --- |
|  Turn AC power switch ON | display lights, Fault |
|  Press RESET |  Ready |
|  select VOLTS  Press and turn Adjust CCW | @Display, set minimum voltage |
|  select FREQUENCY  press START |  Heat On @Display, record FREQUENCY |
|  press STOP |  |

The power supply has scanned the heat station and coil and found the resonant frequency for that combination. Use this figure to determine the optimum series inductor tap for your system.

 Refer to the table in the Installation Drawings for [64] and [67] wire tap selection

 If a change is indicated, remove power and open the power supply; make the changes.

 Replace, close and secure the cover, restore water and AC power,

Transformer Settings

While the power supply output voltage is adjusted from the front panel, selection of the output voltage range is accomplished using the RF Transformer taps. This ability may be required to optimize your loading.

| Do this... | Observe... |
| --- | --- |
|  Insert your work-piece  Press START, select VOLTS |  |
|  Press and turn Adjust CW to 30% | @Display: 30% VOLTS |
|  Select AMPS | If AMPS is more than 30%, refer to tap-setting table; select a lower voltage setting and re-try. Otherwise, resume. |

Do not exceed maximum heat station voltage (§9.4)

|  Select VOLTS  Press and turn Adjust CW to 100% | @Display: 100% VOLTS, or until desired temperature at your work-piece |
| --- | --- |
|  Select AMPS | If work-piece is not hot enough at 100% VOLTS, refer to tap-setting table; select a higher voltage setting and re-try from 30% VOLTS setting If AMPS (%) fails to follow VOLTS (%), reduce series inductance. If you experience PM1 under 100% VOLTS, increase series inductance. |

## When a Fault Occurs

If your power supply senses a problem in the way it’s operating, a fault is imposed:

- at the front panel, you see Fault

- A fault code is stored in the machine’s memory

- RF voltage generation is halted, heat is turned off

- START is disabled until the fault is corrected

Usually, control returns to Ready as soon as the condition is corrected (e.g., water flow restored to an enabled FLS) and you are free to START heating.  In select safety-related cases, however, you must press RESET after the fault has been cleared (see blocked codes in table, next page) before Ready can be re-established. Codes PSOV and PSOC require that the Power Supply is turned off and on again before proceeding.

To read any faults NovaStar has stored, use STATUS; any active fault codes (conditions that are in effect now) are declared as the display alternates between

NOW  CODE

This will persist as long as Fault is shown on the front panel.  The system can display several faults at once.  For example, when you first turn on the power supply and before you press RESET, notice the (STATUS) display:

NOW - PSFS – PSPH

This is the normal condition since the circuits associated with these faults have not been powered yet!  After RESET — and if all safety circuits are OK — these faults are cleared and STATUS shows:

OK

NovaStar keeps a record of the previous fault codes; whenever a fault condition is corrected, it is displayed when you press the Adjust dial while STATUS is selected..  In this power-up example, RESET has been successfully enacted, STATUS shows nothing and when you press Adjust you see

WAS – PSFS – PSPH

Further, whenever control moves from Fault to Ready, display is cleared and NOW codes are moved to WAS.

| CODE | This means... |  | How to proceed... |  |
| --- | --- | --- | --- | --- |
| NOW | Most recent code to follow |  | - - - |  |
| WAS | Previous code to follow |  | - - - |  |
| OK | No codes to report |  | - - - |  |
| Safety circuitry codes |  |  |  |  |
| FLS1 or FLS#n | FlowSwitch#n is enabled and OPEN (1 to 5 FlowSwitches) |  | Identify FLS#n, assure flow is adequate to close switch |  |
| PSDR | PS interlock-switch is OPEN |  | Install cover, tighten screws |  |
| HSDR | HS interlock switch is OPEN |  | Close HS door/barrier or secure HS DOOR jumper |  |
| EMST | FP E-Stop is depressed |  | Turn/clear FP E-Stop |  |
| RMST | Cust. or Pend. E-Stop depressed OR any remote device in loop open |  | If used: clear remote E-Stop or other device. If not used: secure CTB2 jumpers |  |
| PCBoard-to-board communication codes |  |  |  |  |
| TUCE | Tuner communication error |  | Turn PS ON/OFF, retry; Check all ribbon cable connections (A,B,C,E,F) or call service |  |
| FPCE | FP communication error |  |  |  |
| BPCE | BP communication error |  |  |  |
| PRES | MEB - FP communication error |  |  |  |
| Resonant or user-input (START-STOP) frequency codes |  |  |  |  |
| FRHI | Resonant freq.above tuning range |  | Examine HS/coil/work piece/Sense PCB/RF x-former Check for shorts or loose connections. |  |
| FRLO | Resonant freq. below tuning range |  |  |  |
| FROR | Frequency out of range |  | Examine sensing & RF cables for loose connections. Check Power Module fuse(s). Sense PCB disconnected? |  |
| FRRG | Frequency difference too small |  | Difference must be 50kHz or more |  |
| FRUP | Frequency above machine range |  | Check START, STOP freq; re-enter |  |
| FRDN | Frequency below machine range |  |  |  |
| FRGR | Upper, lower frequencies reversed |  |  |  |
| FROP | Invalid frequency entered |  |  |  |
| FCCR | Freq. within FCC prohibited band |  |  |  |
| FCCH | High freq within FCC band |  |  |  |
| FCCL | Low freq within FCC band |  |  |  |
| NOCT NOVT | Sensing or RF cable disconnected or work-loading is too low |  | Examine RF cable: Secure at HS and at PS? Sense cable (CTB3) connected? Work-piece indexed? Coil shorted? |  |
| PM1 or PM#n | PowerModule #n: module fuse(s), over-temperature or over-current |  | Turn PS OFF, cool 5-10min, lower VOLTS, retry, check cable D, check #16-17 plug @ RF module. See InstnDwg for RFXformer settings or call service |  |
| Internal Voltage regulator codes |  |  |  |  |
| PSFS PSPH | (both: normal before RESET) or regulator fuse FU1 Missing phase sensed or FU1 open |  | Press RESET. If persistent: CTB2 connected? PSDR HSDR too? Close PS, HS covers, retry Check 3 wiring, check VReg FU1 or mains FU1 |  |
| PSCE | Comm error from regulator |  | Turn PS ON/OFF, retry; check cable F or call service |  |
| PSNO |  |  |  |  |
| PSDT | Discharge resistor overtemp detected |  | If discharge enabled, duty-cycle >5Hz? |  |
| PSHT | Heatsink overtemp detected |  | Turn PS OFF, cool; check water temps |  |
| PSOC | Internal overcurrent detected |  | Turn PS OFF/ON, lower VOLTS, retry; check AMPS < maximum, wires#29,20 shorted? or call service |  |
| PSOV | Internal overvoltage detected |  |  |  |
| SYNT | Frequency synthesizer error |  | Call service |  |
| PS=PowerSupply |  | HS=HeatStation | FP=FrontPanel | RP=RearPanel |
| CODE | Blocked codes must be followed by RESET after correction of condition |  |  |  |

# Customizing Your System

NovaStar is versatile in the way it’s controlled:

- Do you want to control START / STOP manually or automatically (§6.2)?

- Will you operate it manually while standing near the Heat Station (§6.3)?

- Does your process require power, current or voltage to be held constant?

- Do you prefer serial control and data acquisition of your process?

- Do you require one-button control of heating?

## Changing Setup Switches

The NovaStar system gives you on-demand control of these configuration options with two banks of eight switches. The label on the rear panel (shown) describes what each switch or switch-pair does.

Start type

One-button () means Heat On occurs when the START button is pressed and ceases when the START button is released. If rear panel controlled, Heat On commences when 24Vdc is applied to START (+) at CTB1 and ceases when 24Vdc is removed.

Two-button () means Heat On commences when the START button is pressed and remains on when released. Heat On ceases when STOP is pressed (front or rear).

Start from

Rear panel: your PLC signal or a pendant is used to START; connections are made at CTB1. When selected, START can only be initiated from the rear panel. STOP is always active at the front and the rear.

RS-485: serial control using a PC and the control command set. When selected, START can be initiated only from the serial port. STOP is always active at the front and the rear.

Front panel: when selected, the buttons on the power supply control START and STOP. Rear panel and RS-485 control of START are disabled.

Input from

Rear panel: CTB2 accepts either your optional voltage or current source (see ‘Source’, below) which is calibrated to provide your desired output level (§6.4)

RS-485: serial control using a PC. Your command-string (§8) instructs the power supply to hold the output to the specified level (V, A or W).

Front panel: SELECTOR and Adjust are used to pre-set the required output level (V, A, W)

Output Regulation

VOLTS: the output is controlled according to the dc voltage generated by the power supply. With this option, you specify (from the front panel, serial port or rear panel) a voltage setting; the supply adjusts the Heat On level to meet that voltage.

When the front panel is selected (Input from, above) the Adjust dial sets the output level. When you select RS-485 control, a command string sets the output level. When you select output control from the rear panel, an analog voltage or current source controls the output.

Input signal type

Your analog source (voltage or current) is used to control the output. This applies only when INPUT FROM is set for the rear panel. Several source options (§6.4) are selectable.

Reset from

Your power supply must be RESET every time you power it up and following certain safety-related Faults. This setting determines where the power supply looks for the RESET signal.

Standard configuration

This is the setup to use to control START, RESET and the tank voltage from the front panel of your NovaStar: During Heat On, the system holds the internal DC voltage constant, while tank voltage may change with loading.

| Sw# |  | Function | Effect... |
| --- | --- | --- | --- |
| 1 |  | START TYPE | Selects 2-button operation |
| 2 3 |  | START FROM | Selects front panel |
| 4 5 |  | INPUT FROM: | Selects front panel |
| 6 7 |  | OUTPUT REGULATION | Selects DC voltage |
| 8 |  | Not declared: | Must be set in this position |
| 9 10 |  | INPUT SIGNAL (rear-panel control only) | Doesn’t matter; control is from front panel |
| 11 |  | RESET FROM | Selects front panel button |
| 12 13 14 15 16 |  | Not declared | Must be set in this position |

Control from serial port (RS-485)

The RS-485 serial port is always active (see §8); this setting enables START and OUTPUT to be controlled from your computer.

| Sw# |  | Function | Effect... |
| --- | --- | --- | --- |
| 1 |  | START TYPE | Selects two-button operation |
| 2 3 |  | START FROM | Selects serial port (RS-485) |
| 4 5 |  | INPUT FROM: | Selects serial port |
| 6 7 |  | OUTPUT REGULATION | Selects DC voltage |
| 8 |  | Not declared: | Must be set in this position |

Control from rear panel

This setup permits you to connect your (optional) pendant controller (§6.3):

- START & STOP are controlled from the pendant,

- Output is controlled by voltage using the front panel to set the desired voltage.

- RESET is controlled from the front panel

The state of your system will continue to be displayed at the front panel but will also be shown at your pendant.

| Sw# |  | Function | Effect... |
| --- | --- | --- | --- |
| 1 |  | START TYPE | Selects two-button operation |
| 2 3 |  | START FROM | Selects START from rear panel |
| 4 5 |  | INPUT FROM: | Selects front panel; use VOLTS and Adjust to set level |
| 6 7 |  | OUTPUT REGULATION | Selects DC voltage |
| 8 |  | Not declared: | Must be set in this position |

Control from rear panel with 4-20mA

This setup enables you to connect your isolated 4-20mA controller source. The power supply scales its output from 0 to 100% VOLTS (in this example) based on the current level from your controller.

- START, STOP and RESET are controlled from the front panel.

| Sw# |  |  |  | Function |  | Effect... |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | START TYPE |  | Selects two-button operation. |  |
| 2 3 |  |  | START FROM |  | Selects front panel |  |
| 4 5 |  |  | INPUT FROM: |  | Selects rear panel |  |
| 6 7 |  |  | OUTPUT REGULATION |  | Selects DC voltage |  |
| 8 |  |  | Not declared: |  | Must be set in this position |  |
| 9 10 |  |  | INPUT SIGNAL (rear-panel control only) |  | Output Control from rear panel, selection is applicable to a 4-20mA current source |  |
| 11 |  |  | RESET FROM |  | Selects RESET from the front panel button |  |
| 12 13 14 15 16 |  |  | Not declared |  | Must be set in this position |  |

## Choosing Start and Reset Options

There are several ways to command your power supply to begin heating: 1- or 2-button, front or rear panel controlled or serial controlled. §6.1 illustrates several switch settings required to enable these options; this section explains their application and installation.

START/RESET from front panel

This configuration expects you to control your process from the front panel of the power supply. No additional installation is required; START, STOP and RESET are all active. The Switches #2 and #3 must be set as shown.

| 1 |  |  | System expects START control from the buttons on the front of the unit. |
| --- | --- | --- | --- |
| 2 |  | front panel control |  |
| 3 |  |  |  |

| 1 |  | one-button | Once Ready, you must press START and hold it in order to keep Heat On; heating takes place only while you are holding the button.. |
| --- | --- | --- | --- |
| 2 |  |  |  |
| 3 |  |  |  |
|  |  |  | Heating halts either when you release START or (if enabled) when the timer has expired. |

| 1 |  | two-button | Once Ready, you must press START momentarily for Heat On. You must then press STOP to halt heating; power is delivered to the Heat Station until STOP is pressed. |
| --- | --- | --- | --- |
| 2 |  |  |  |
| 3 |  |  |  |
|  |  |  | If you have enabled the timer, heating will halt when the timer expires or when STOP is pressed. |

| 9 |  | front panel RESET | RESET must be pressed each time you turn on your power supply and following certain safety-related Faults. This setting enables the front panel RESET button for this purpose. |
| --- | --- | --- | --- |
| 10 |  |  |  |
| 11 |  |  |  |

START/RESET from rear panel

If you elect to use a pendant controller to operate START, STOP and RESET, you must:

 Complete wiring of the device to the rear panel (§6.3)

 Setup switches accordingly:

| 1 |  |  | System expects START control from CTB1, at the rear; you will connect controlling hardware. |
| --- | --- | --- | --- |
| 2 |  | rear panel START |  |
| 3 |  |  |  |

| 1 |  | one-button | Once Ready, you must press START at the pendant and hold it in order to keep Heat On; heating takes place only while you are holding the button. Heating halts either when you release START or (if enabled) when the timer has expired.. |
| --- | --- | --- | --- |
| 2 |  |  |  |
| 3 |  |  |  |

| 1 |  | two-button | Once Ready, you must press START momentarily at the pendant to keep Heat On. You must then press STOP to halt heating; power is delivered to the Heat Station until STOP is pressed. If you have enabled the timer, heating will halt when the timer expires or when STOP is pressed |
| --- | --- | --- | --- |
| 2 |  |  |  |
| 3 |  |  |  |

| 9 |  |  | System must be RESET each time you turn on your power supply and following certain safety-related Faults. N.O. pendant switch or your 24V PLC signal is expected at the rear panel. |
| --- | --- | --- | --- |
| 10 |  | rear panel RESET |  |
| 11 |  |  |  |

START from serial port

If you elect to use your RS-485 port to operate START and STOP, you must:

 Complete wiring of the serial device to the rear panel (see §6).

 Setup switches as shown SW1, SW2

| 1 |  |  |
| --- | --- | --- |
| 2 |  | RS-485 START |
| 3 |  |  |

RESET is not controlled over the serial port; you must first clear the causing Fault then manually RESET the Power Supply.

Make sure SW11 is correct for your RESET location, front or rear panel.

## Connecting Your Pendant

In order to control your NovaStar from a remote pendant controller, you must:

- set switches as needed

- complete wiring of the controlling hardware

One button START

 Set SW1-SW3 as shown. §6.1 explains why this results in:

| 1 |  | one-button |
| --- | --- | --- |
| 2 |  | rear-panel control |
| 3 |  |  |

 There are 2 minimum connections that you must make:

- 24Vdc

- START

Ameritherm offers pendant controllers; the wire numbers for connection to these controllers is [bracketed] in this instruction.

This configuration (upper illustration) uses the internal 24V(+) to trigger START, either through your N.O. switch or through your optional Ameritherm pendant. Since one-button control is selected, Heat On persists only while START is depressed. The STOP jumpers must be retained since the system will halt if these two signals are interrupted.

An alternative to this minimal arrangement (lower illustration) uses your 24Vdc source to trigger START, and (assumes) your external switching:

Again, Heat On persists only while the switch is depressed. Your isolated 24Vdc supply is wired as shown and the internal 24V supply is not used

.

Two Button Start

If your process requires that Heat On persists until a subsequent signal or a timer intervenes, two-button control of START is available

 Set SW1 as shown (see §6.1)

| 1 |  | two-button |
| --- | --- | --- |
| 2 |  | rear-panel control |
| 3 |  |  |

 Complete wiring of the hardware per:

2-button control allows Heat On to persist until STOP is pressed, timer expires or Fault occurs. Notice that, following removal of the positive jumper, the STOP signal is provided from the N.C. switch in the pendant. (STOP is always active from the front and the rear panel; you can still interrupt Heat On from the front panel while configured like this.)  The negative jumper is still required at START and STOP since this diagram only switches the positive portions.

In order to use your 24V signaling (lower illustration), complete the wiring from your equipment as shown. Note that the STOP jumpers are removed completely, since your equipment provides the required 24V and 0V references.

Remote RESET connection

If you wish to make use of an N.O. switch on your remote or pendant in order to RESET the system, you must:

 Set switches:

| 1 |  | two-button START |  | 9 |  | doesn’t matter: INPUT from front |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  | rear-panel START |  | 10 |  |  |
| 3 |  |  |  | 11 |  | RESET from rear |
| 4 |  | input from front |  | 12 |  | required setting |
| 5 |  |  |  | 13 |  |  |
| 6 |  | regulation of voltage |  | 14 |  |  |
| 7 |  |  |  | 15 |  |  |
| 8 |  | required setting |  | 16 |  |  |

 Complete wiring of the hardware per:

It’s important to notice that START, STOP and RESET each require a ground reference as well as a 24Vdc signal, accomplished in this diagram with an added jumper.

To use your 24V signaling hardware, RESET is another N.O. signal to 24Vdc:

Complete pendant connection

This diagram demonstrates connection of a complete pendant controller (yours may vary in hardware or wire number); follow this example when completing your installation. The indicator lamps are not required for operation, but they are recommended in order to ensure that operators are informed of the state of the equipment when they may not be able to see the power supply

Remote E-Stop connection

At the rear panel, there are four terminals on CTB2, provided for the connection of:

Pendant Emergency-Stop

Customer Emergency-Stop

These terminals expect a normally closed (N.C.) device, so, typically, two jumpers are factory-installed. You must remove the jumpers to attach your E-Stop(s).

E-Stop acts during Ready or Heat On, establishing Fault when the switch opens. Supply voltage is then interrupted to most of the internal circuitry and RF synthesis is halted. Once the reason for the E-Stop has been determined and corrected, you must clear the E-Stop switch and press RESET; the system will re-check the safety circuits and establish Ready if everything is OK.

E-Stop is always enabled (front and rear panels); no switch or serial setting is required.

If you are connecting only one E-Stop, make sure the other jumper remains in place! Without it, you won’t be able to clear Fault.

## Setting Output Control Options

For this, you plan to use your temperature controller or PLC to set the output level of the NovaStar system to correspond to your analog output (voltage or current). To accomplish this configuration:

 Review your control equipment literature; determine the output:

- Voltage, 0-5Vdc or 0-10Vdc?

- Current, 0-20mA or 4-20mA?

 Setup switches accordingly: (This example: 4-20mA source)

Current controlled

| 1 |  | two-button START |  | 9 |  | System expects your 4-20mA source (this example, see §6.1) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  | rear-panel START |  | 10 |  |  |
| 3 |  |  |  | 11 |  | RESET from front panel button |
| 4 |  | INPUT FROM rear panel |  | 12 |  | required setting |
| 5 |  |  |  | 13 |  |  |
| 6 |  | regulates DC voltage |  | 14 |  |  |
| 7 |  |  |  | 15 |  |  |
| 8 |  | required setting |  | 16 |  |  |

Voltage controlled

| 1 |  | two-button START |  | 9 |  | System expects your 0-10Vdc source (this example, see §6.1) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  | rear-panel START: |  | 10 |  |  |
| 3 |  |  |  | 11 |  | RESET from front panel button |
| 4 |  | INPUT FROM rear panel |  | 12 |  | required setting |
| 5 |  |  |  | 13 |  |  |
| 6 |  | regulates DC voltage |  | 14 |  |  |
| 7 |  |  |  | 15 |  |  |
| 8 |  | required setting |  | 16 |  |  |

## Connecting Your PLC

NovaStar is fully controllable from your PLC:

- command signals (START, STOP, RESET) are brought to the rear panel

- system output level is adjustable by remote connection (§6.4)

- states (Heat On, Ready, Fault) are conveyed with isolated relays

The following example assumes

- rear panel (PLC) control of START and RESET

- one-button (trigger) operation of timed (2.5sec) cycle

- 0-10Vdc control of Output Voltage

 Assure that setup switches are correct:

| 1 |  | one-button START |  | 9 |  | System expects your 0-10Vdc source (this example, see §6.1) |
| --- | --- | --- | --- | --- | --- | --- |
| 2 |  | rear-panel START |  | 10 |  |  |
| 3 |  |  |  | 11 |  | RESET from rear; PLC signal |
| 4 |  | INPUT FROM rear panel |  | 12 |  | required setting |
| 5 |  |  |  | 13 |  |  |
| 6 |  | regulates DC voltage |  | 14 |  |  |
| 7 |  |  |  | 15 |  |  |
| 8 |  | required setting |  | 16 |  |  |

 With the power off, complete the wiring to your PLC as shown; all digital inputs must be 24Vdc (16Vdc min) and must be able to source 7mA minimum.

 At the front panel (with power on), select TIMER

 Press and turn ADJUST clockwise until the display reads 2.500. Each following heat-cycle will persist for this period until you reset the TIMER to 0.000

All digital I/O are isolated to 2500V as long as you do not include equipment ground (0Vdc terminal,CTB1). Equipment operation is not affected if you elect to use the internal 24Vdc source for these signals, but isolation is forfeited

Refer to the timing diagram to sequence your control I/O.

Timing diagram

| tflt | ~5000 | ms |
| --- | --- | --- |
| trst | 20 |  |
| tchk | ~5000 |  |
| tstrt | 20 |  |
| trise | 153 |  |
| ttune | 25-401 |  |
| ton | 100 |  |
| tfall | 40-1502,3 |  |
| toff | 10 |  |

1) 25-65 ms for NS3 High Freq

2) for 25ms max: specify Rapid Discharge option

3) Each Output Voltage edge can display  5ms dither from software loop.

# Theory of Coil Design

remove this page and replace with 801-9038

# Serial (RS-485) Control

NovaStar can be controlled from a serial communications port. This means that with Ameritherm’s optional RS-232 to RS-485 converter kit and a little software (see below) you can control your process remotely and store periodic operating data pertaining to the process. Remotely, you can:

- START and STOP heating

- Adjust Heat-cycle duration

- Control output by VOLTS, AMPS or POWER

- Collect key operating data on demand

- Calibrate primary input and output values

What’s RS-485?

RS-232 is the name given to the serial communication rules controlling how personal computers communicate with nearby cabled devices. This method can become unreliable at separations over 50 feet. For this reason, NovaStar uses a hybrid RS-422/485 full-duplex protocol.  Single- or multi-drop configurations at separations over 1000 feet are expected; you can control several NovaStar units from one central PC!

What do you need to use RS-485?

To control and monitor your NovaStar over this serial COM link, you will need:

- a computer (PC)

- serial communications software

- an RS-485 serial port OR

- (optional) RS-485 interface converter

- cabling terminated with d-sub 9 male hood for each power supply

Serial Communications Software

You can use any of a number of communications software packages such as:

- Procomm

- HyperTerminal

The following example uses HyperTerminal running on the Microsoft Windows95 operating system. Confirm that HyperTerminal has been installed on your system and…

 Use Start  Programs  Accessories  HyperTerminal to open this window:

The window you see may be somewhat different; just look for the HyperTerm icon and…

 Double-click the Hypertrm.exe icon

The dialog box you see will ask you to ‘name’ your connection. Once you complete this procedure, the named connection will be available for use without requiring this naming and setup procedure.

 Enter a name for your connection (example: NovaStar). Select an icon for this connection and click OK.

The name and icon you’ve selected will – after completion of this procedure – appear in the HyperTerminal window.

The next dialog box asks about the hardware you’ll be using to make your connection. This (typical) example uses COM1, the standard serial port.

 Click the down arrow in the “Connect using” option-box; select “Direct to Com 1.” Click OK.

Obviously, you will not require a telephone number or country code in order to connect to you NovaStar power supply.

During this setup procedure, you must tell HyperTerminal the communications ‘rules’ it must follow to talk to NovaStar:

 Click the drop-down arrow at each option box; make the selections as shown. Click OK

When this dialog box closes, HyperTerminal’s main screen is displayed, mostly blank. There are a couple steps you must perform to complete the procedure:

 From the HyperTerminal toolbar, click

File   Properties. A new dialog box appears with two tabs at the top.

 Click Settings to display:

 Click the drop-down arrow and select VT100 from the option box.

 Click the ASCII setup button to open the next dialog box:

These selections have to do with the way the communication between HyperTerminal and the NovaStar power supply are displayed on your screen

 Click the check-boxes as shown; click OK

 Click OK to exit the Properties dialog box; this returns to the HyperTerminal main screen.

With Windows 98, there is a bug in HyperTerminal in which the ‘Echo typed characters…’ button doesn’t work; you do not see the characters you’ve typed on-screen. You can get a free fix by going to www.hilgraeve.com.

You may exit HyperTerminal now; when you re-open the application, your NovaStar setup icon will appear in the window. You can drag it to your desktop to save the steps of opening HyperTerminal.

COM Port/Interface Converter

Ameritherm provides an option kit containing all the hardware you need to link your PC to the NovaStar supply:

- RS-232 to RS-485 interface converter

- 25’ cable to the supply

- adapter hardware

If you have ordered this option kit to link your PC to the supply, you must:

 assemble and connect the converter as instructed.

 attach the 25’ cable to the converter following the numbering or instructions

 attach the 9-pin hooded connector to the supply; secure the jack-screws

 Plug the adapter transformer to 110Vac; insert the other end into the converter.

Configuration of your PC/converter must follow:

| Data device type | Comm Mode |
| --- | --- |
| DCE | TxON/RxON |

Pin-out is per:

| Power Supply pin | 4 | 5 | 8 | 9 |
| --- | --- | --- | --- | --- |
| Function | TX+ | TX- | RX- | RX+ |

Password

As shipped, your NovaStar does not require a password to use any of the commands; you can use all the commands on the next page any time.

If you need to restrict serial operation of the equipment, set pass[ID],on. The next time the NovaStar is powered-up, only the commands at the top of the list, right, will be available without the password. Use of all the other commands in the list is enabled with the next pass[ID],cust command or by returning to pass[ID],off, as the unit (typically) left the factory.

Command Set

This table gives examples of each command’s use and the range of the data you can use in the command’s ‘argument’.  The comma between the command and argument is required; do not insert spaces or brackets! Each command must be arranged like this:

command[ID],[argument]

the ‘setting’ to be used; unique for each command

000 thru 127; individual device ‘addresses’

or 00A thru 00M; gang addresses

instructions; from the following table

Parentheses indicate the default/factory state or condition.

| Command | Purpose |  | Example | Argument | Notes |
| --- | --- | --- | --- | --- | --- |
| r[ID] | Read operating values now |  | r001 | none |  |
| n[ID] | Names the unit [ID] |  | n004 | 000 - 127 | single-unit only |
| pass[ID],[arg] | When enabled (PASS001,ON) input required RS-485 password |  | pass007,cust | cust or on / (off) | (default) password not required |
| setup[ID] | Read, display all equip. settings |  | setup003 | none |  |
| The following commands available with CUST password |  |  |  |  |  |
| h[ID],[arg] | START / STOP Heat cycle |  | h001,on | on / off | Switches set: #2 #3 |
| h[IDg],[arg] | Gang addressing; simultaneously START / STOP Heat cycle for groups of up to 10 units (table) |  | h00a,on |  |  |
| v[ID],[arg] | Set Voltage % regulation min 5 thru max 100 |  | v004,50  TV, below | min / max | Switches set: #4 #5 |
| t[ID],[arg] | Set heat cycle timer |  | t001,23 | 9999, max 0000 = OFF | also set from front |
| tv[ID],[arg] | Set tuning voltage %; min 5% |  | tv002,20 | (min) - max |  |
| %[ID],[arg] | Dis/enables front panel V, A, W display as percent-of-full-scale |  | %004,on | off / (on) | units V, A, W |
| fault[ID] | Read Fault record(s) now |  | fault003 | none | fault-time op values |
| fls[n][ID],[arg] | Dis/enables monitoring of flowswitch inputs |  | fls3002,on (FLS3; unit 002) | (off)/on n = 1-5 | FLS6 and FLS7 reserved; do not use |
| Following commands active only if OUT1 and OUT2 are enabled as a factory option. |  |  |  |  |  |
| do1[ID],[arg] |  | Set OUT1 state via RS-485 | do1001,on | on / (off) | on = closed off = open |
| do2[ID],[arg] |  | Set OUT2 state via RS-485 | do2001,on |  |  |
| do1[ID], fls[n],[arg] |  | OUT1 state tracks FLSn state | do1001,fls2,on | on / (off) | en/(dis)able feature FLSn, n=1-5 |

Controlling multiple units

You can connect several NovaStar units together on the same RS-485 control ‘bus’; since different ID numbers are used, each unit responds only to those commands that have its ID number.

 Contact Ameritherm for information about the option kit and instructions used to link multiple units

Gang Control of START / STOP

(For multiple-unit RS-485 control and for the h command ONLY)

If you have linked several units for control under RS-485 (see above), NovaStar3 provides a way to simultaneously START and STOP heating by a group of power supplies.

Using ‘gang’ ID addressing, up to ten units are commanded to START (or STOP) simultaneously; the gang ID is two zeroes followed by a capital letter from A to M:

| ID range | gang address | ID range | gang address |
| --- | --- | --- | --- |
| 000 – 009 | A | 070 – 079 | H |
| 010 – 019 | B | 080 – 089 | I |
| 020 - 029 | C | 090 – 099 | J |
| 030 – 039 | D | 100 - 109 | K |
| 040 – 049 | E | 110 - 119 | L |
| 050 – 059 | F | 120 - 127 | M |
| 060 – 069 | G |  |  |

 In gang, it’s best to ignore the password requirement (default)

 Using the n command, individually connect to and program your seven units (for example) IDs 001 through 007. Never try to change IDs while you are connected to more than one unit.

 Re-establish your multiple-unit RS-485 connections to the seven units

 Upon Ready, type h00A,on; this will have the effect of STARTing the seven units, since their IDs fall within the 00A gang.

 Observe each unit’s response; Hon1 confirms Heat On for unit 001, etc.

 Type h00A,off, see each unit’s confirmation: Hof1, Hof2, etc

Example

This is an example of a typical serial control sequence. To begin:

the serial cable has been connected at the power supply and the PC

the PC’s communication software is running, password is ON.

switches #2, #3, #4 and #5 are set as shown:

| Sw# |  | Function | Effect... |
| --- | --- | --- | --- |
| 1 |  | START TYPE | Selects two-button operation |
| 2 3 |  | START FROM | Selects serial port (RS-485) |
| 4 5 |  | INPUT FROM: | Selects serial port |
| 6 7 |  | OUTPUT REGULATION | Selects DC voltage |
| 8 |  | Not declared: | Must be set in this position |

1

3

4

5

6

7

2

1 When you hit ENTER at first, the power supply ‘hears’ the PC and acknowledges with Err1. Since password is enabled, a password must be entered.

2 Your power supply, by default, has been given the [ID] 001, your password is cust. The system has approved your password and is ready for your next command

3 The r001 command tells the system to display the present operating values. This unit is Ready. The frequency range for the equipment in the example ends at 485kHz. The timer is disabled and there have been 4 heat cycles. The last number is a checksum.

4 To elevate the output voltage to heat your part, use the command v over the RS-485 link to set the Heat On voltage. This example sets 50%.

5  h001,on tells the power supply to apply RF power to the Heat Station and the work-piece. Since the timer is disabled (0.0 sec), Heat On will persist until STOP is pressed or until the h001,off command is sent.

6 The r001 command during Heat On reflects the higher voltage and indicates that there is significant current flow. Cycle count is 5

7 h001,off  is the serial command to turn off the heat.

FAULT command

NovaStar communicates its fault record — both NOW and WAS (§5.2) — with this command.  In addition, the volts, amps, frequency and the state (Ready, HeatOn) at the time the fault occurred are all displayed.

Data timing

Transmissions are 9600 baud: 10-bits/character = 1.04ms/char. Each string is delimited with 4 non-printing control characters (4.2ms). Typical auto-output string is shown; n represents any numeral. Checksum (in this example, 27) is hexadecimal.

(CR)(LF)nnn%V_nnn%A_nnn_KHZ_nnnn_sec_nnn_cyc_27(CR)(LF)

# Appendices

This section organizes several important service and related topics.

## How to Replace Fuses

You may have to replace fuses if:

- your system does not respond when you apply power

- the code PSPH (missing phase detect) is displayed

- Output port not responding (Ready, Heat On, Fault, etc)

No Response to AC Power

 Turn off all AC power to the system, including your fused disconnect

 Remove the cover as described in §3.

 Locate the fuses in the front of the power supply

 With a DMM ( setting) check continuity of each fuse.

 Replace any fuse reading open (>10); see the chart for replacements.

If PSPH is displayed

 Make sure all AC power is off!

 Examine FU1. Replace it if it’s opened.

 If it’s OK, look at the soft-start PCB; FU1 on that circuit that may have opened.

If Output Port is not responding

 Locate the rear panel output fuse for that port

 Pull out the two-pin PCB-mounted fuse

 With a DMM ( setting) check continuity of the fuse.

 Replace any fuse reading open (>10); see the chart for replacements.

Equipment fuses

|  |  |  | 220Vrms | 440Vrms | 220Vrms | 440Vrms |
| --- | --- | --- | --- | --- | --- | --- |
| Mains | FU1, FU2, FU3 | 13/32x1.5” | 15A600V | 10A600V | KLDR 15 CCMR 15 LPCC 15 | LPCC 10 ATDR 10 KLDR 10 |
| Control wiring | FU7, FU8, | 13/32x1.5” | 4A600V |  | ATQR 4 |  |
|  | FU9 |  | NA | 4A600V |  |  |
| PCB Soft-Start | FU1, FU2, FU3 | 13/32x1.5” | 0.5A600V |  | KTK ½ ATMR ½ |  |
| PCB VReg | FU1 | 13/32x1.5” | 15A600V |  | KLM 15 KLKD 15 ATM 15 |  |
| PCB RFPow | FU1 | 13/32x1.5” | 20A600V |  | KLKD 20 KTK 20 ATM 20 |  |
|  | FU2 |  | 1A250V |  | TR5-T |  |
| Outputs | rear panel 6x |  |  |  |  |  |
| 24V PSupply |  |  |  |  |  |  |

European distributors of fuses used

| Little | Bussmann | Country |
| --- | --- | --- |
| Littlefuse UK, Ltd. Ph 191-4158181 | Kempston Electrical Co, LTD Shirley Road Rushden, Northants | England |
| Anatronic, SA Paseo Imperial 8 Planta 3-3B 28005, Madrid | BFI IBEXA Electronica SA Edificio ALFA III, Nave 8 Poligono Ind Fuencarral 28049 Madrid | Spain |
| Nores Oy Mankkaantie 32 02180, Espoo | Perel, GV POBox 230 Torpankatu 28 FIN-05801 Hyvinkaa | Finland |
| Elis Elektro A/S Jerikoveien 16 Lindeberg Gard, N-1007, Oslo | Arthur F Ulrichsen AS Hasleveien 28 Oslo, 5 | Norway |
| Altex POBox 85124 Sundsvall | Visitron Metallvagen S-690 45 Asbrow | Sweden |
| R.D.I. 5-7 Allee Louis Breguet Zac de Villepinte, 93421, Villepinte | Arcel 54 Rue de Vincennes 93100 Montreuill | France |
| Orvem SPA Via Domodossola 17 20145, Milano | Fusimport Via Gluck 50 20125 Milano | Italy |
|  | Kromberg and Schubert Ireland Unit 31 Industrial Estate, Waterford | Ireland |
| Michel Decamp NV Avenue de l’Artisanat 10 B-1420, Braine-l’Alleud | MCA tronix Parc Scntf du Sart-Tilman Aves Des Noisetiers, B-4031 Angleur | Belgium |
| Anatronic Aveiro Rua Marquez Gomez Edif. Vera Cruz/6 Dt0. Tras 3800, Aveiro | Cablesa Ind De Comp Zona Ind Montalvao 6000 Castelo Branco | Portugal |
| Eurodis Electronics AG Bahnstrasse 58/60 CH 8105, Regensdorf | EME AG Lohwisstrasse 52 CH8123 Ebmatingen | Switzerland |
| Eltax GMBH Blechfurmgasse 13, A-1050, Wien | Lipschitz & Co GESMBH Felbigergasse 38 A-14140 Wirn | Austria |
| R Hirschmann BV POBox 92 1380 AB, Weesp | Cisper Electronics Koperslagerij 32 4762 AP Zevenbergen | Netherlands |
|  | Gemballa Eltns GMBH c/o Erik Halskov Jensen GMBH Neustadt 10 D-24939, Felnsburg | Germany |
| Berensen Eltech Telefonvej 8 DK-2860, Soborg |  | Denmark |
| Ypco Electronics 13123 Niliosia Athens |  | Greece |

## How to Order Replacement Circuit Boards

Inside the power supply, the MEB is calibrated with the Rear Panel and with the Voltage Regulator PCB. If it is ever necessary to replace any of these three PCBs, a re-calibration should be performed before returning the repaired unit to service (a little like balancing new tires or re-setting your VCR time). Use of un-calibrated PCBs can result in reading errors of not more than 5%. Call Ameritherm to discuss your options to complete this re-calibration:

- return unit to Ameritherm for PCB replacement and calibration

- purchase calibrated PCB sets

- have Ameritherm conduct calibration on your premises

- request procedure from Service and calibrate new PCBs yourself

## How to Clean the Air Filter

Your air-cooled NovaStar filters the air used to cool the internal components. This filter must be removed and cleaned periodically to assure the best heat-exchange and optimum system performance. We recommend cleaning every six months but, depending on your area, you may need to repeat this process on a monthly basis.

 Turn off the AC power switch..

 Turn off the customer-supplied fused disconnect; after this step, all power is removed.

 At the right side of the NovaStar, use a philips screwdriver to remove (4) screws securing the side vent.

 Remove the vent/filter assembly and slide the air filter out of the slots.

 Use either a vacuum cleaner or compressed air to free the loose dirt from the inlet side of the filter. You may also soak the filter in warm soapy water for 15-20 minutes. Follow this with a compressed air drying against the direction of the arrows.

 Place the filter in slots; secure the assembly using (4) screws previously removed.

Note flow arrows on the edge of the filter frame; align these arrows to point into the power supply when you re-assemble it.

## Heat Station maximum ratings

Your Heat Station can be damaged by the application of excess RF voltage. This section – along with the tap-setting procedure (§5.2) – provides enough information to help you ensure this limit is not exceeded.

The maximum RF voltage levels for your specific heat station configuration (shipped from Ameritherm) are highlighted in yellow in the table at the end of this section.

This procedure uses the table to determine the maximum RF transformer tap settings:

 Read your heat station data tag; find configuration and total capacitance

1  Find the capacitance reading in the appropriate configuration table, below

 Note the operating frequency of your load combination (@Display while Heat On)

2  Read across the capacitance row to find your operating frequency.

3  Read the maximum RF voltage figure at the top of this column

 Do not set transformer taps higher than indicated by these tables

Parallel configuration

| prisec |  | BA |  |  |  |  | AB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| at taps.. |  | 5:3 | 3:2 | 4:3 | 5:4 | 4:4 | 4:5 | 3:4 | 2:3 | 3:5 | 2:4 | 2:5 |
| max RF volts |  | 100 | 110 | 125 | 135 | 170 | 210 | 225 | 250 | 280 | 340 | 420 |
|  |  | 3 up to this operating frequency (kHz) |  |  |  |  |  |  |  |  |  |  |
| 1 for this total capacitance (F) | 0.2 | 1950 | 1850 | 1800 | 1700 | 1500 | 1300 | 1200 | 1100 | 1000 | 850 | 600 |
|  | 0.27 | 1350 | 1300 | 1200 | 1125 | 950 | 825 | 800 | 675 | 625 | 600 | 500 |
|  | 2 0.34 |  |  |  |  |  |  |  |  |  |  |  |
|  | 0.43 | 1150 | 1050 | 950 | 900 | 700 | 550 | 500 | 425 | 400 | 350 | 275 |
|  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |
|  | 0.66 |  |  |  |  |  |  |  |  |  |  |  |
|  | 0.76 | 725 | 625 | 550 | 525 | 425 | 365 | 335 | 275 | 250 | 210 | 175 |
|  | 0.83 |  |  |  |  |  |  |  |  |  |  |  |
|  | 0.99 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1.32 |  |  |  |  |  |  |  |  |  |  |  |

Example

 Heat station data tag reads: PARALLEL, 0.66μF

 Frequency @Display while HeatOn = 390 kHz

 Table: tap setting 3:5, since 250kHz falls within this column

Note: 2:4 and 2:5 cannot be used; they have max frequency ratings of 350 and 275 kHz, respectively.

## How to Use Matching Capacitors

Under conditions of higher resonant frequencies (fo), coupled with extended RF cable lengths, it is sometimes required to ‘tune-out’ the intrinsic inductance of the unit’s output stage: the RF cable, the RF Power Module, the RF transformer and even the Heat Station. As fo rises, this intrinsic inductance become more significant and can have a negative effect on the unit’s performance.

Based on information we have about your application, we can – if necessary – specify, design and install a ‘nulling’ circuit on your unit while in the factory. This will have the effect of capacitively counteracting this inductance and restoring optimum device performance.  You have to do nothing different in your installation; the circuit is attached to the RF output terminals.

We are happy to discuss this with you as required during our evaluation of your application.

## How to Re-select Line Voltage

If you have purchased a 440Vrms unit, the equipment is designed to accept 3 line voltage at 380, 400, 440 or 480 Vac, 10%. Based on information provided in your order, an internal transformer has been factory-set to meet your line level. (220Vrms units are not equipped with this transformer!)

If you change the AC line voltage level within this range, the internal transformer must be re-tapped. Follow these instructions:

Caution: Stored electrical charge can persist at internal terminals for several seconds following disconnection of AC power! Allow 60 seconds after disconnection before touching or servicing these terminals.

 Turn off the AC power switch..

 Turn off the customer-supplied fused disconnect; this step removes all power.

 Use a philips screwdriver to release ten ¼-turn screws and lift off the cover

 Note location of wire #71 (0V tap); this wire NEVER moves.

 With needle nose pliers, move wire #70 from the factory-setting tap.

 Position wire #70 at the desired tap and press into place with needle nose.

 Close cover and return unit to service.

Insert heat station table after this page