Overview and plan for use of

EPICS Channel Access Protocol on NSCL controllers

National Superconducting Cyclotron Laboratory, MSU

## Created:  Mark Davis, 01/14/2010      Last Updated:  Mark Davis, 01/17/2012

##### EE Document #: ###-####-##-01_00.doc

# Introduction

The majority of embedded controllers developed by the NSCL Electrical Engineering department are based on SBCs (Single Board Computers) that incorporate a RCM (Rabbit Core Module) from the Rabbit Corporation.  The primary means of communication with these devices has been via a ModBus server running on the SBC.  The ModBus protocol was originally chosen because of its simplicity and minimal requirements in terms of memory and CPU usage on the controller.

Communication between the embedded controllers and the EPICS control system is provided by a service running on the EPICS IOCs (Input/Output Controllers) that uses ModBus to talk to the controllers and to several PLCs (Programmable Logic Controllers).  The EPICS records for the embedded controllers depend upon this service to manage most of the communications with the controllers.

Due to limitations in the ModBus protocol itself, there is motivation for adding support for the “native” EPICS CA (Channel Access) protocol.  The expected advantages for such a change include a reduction in development time for new controllers, an increase in performance of the communications link, and the ability to create and maintain a standard for the values that are common to all the controllers.

Project scope

The Rabbit-based controllers at NSCL can be divided into groups along two criteria:  The software platform (DEBROS vs non-DEBROS) and the hardware platform (RCM2200 vs RCM3200).

DEBROS (Davis Embedded Baby Real-Time Operating System) is a small, priority-based, fully-preemptive RTOS (Real Time Operating System) that provides a basic subset of standard Linux/Unix commands and system calls.  It was written and adopted as the basis of all new development for numerous reasons, but the critical one was that we couldn’t add any more code using the previous method and tools.  In contrast, DEBROS has allowed a huge increase in capabilities while actually reducing the amount of memory used.  DEBROS requires the use of the Softools compiler and linker and the SHDesign’s RDLM (Resident DownLoad Manager).

The code for the non-DEBROS controllers was developed using the DynamicC compiler from ZWorld (now part of the Rabbit Corporation).  Due to several reasons (including the much less capable and non-ANSI C compiler and the method used for updating the firmware), we had reached a point where it was not possible to add any significant amount of code to the applications.  Therefore, support for any new capabilities (such as the CA protocol) is limited to controllers running DEBROS, which currently make up about 1/3 of the installed Rabbit-based controllers.  For this reason and many others (such as lack of logging and diagnostic capabilities on the older controllers), porting of the older applications to DEBROS is highly desirable.

On the hardware side, there are two different RCMs (Rabbit Core Modules) in the currently installed NSCL controllers:  The RCM200 and the RCM3200.  The RCM2200 contains a Rabbit 2000 processor clocked at 22.1 MHz and has 512K flash memory and 512K RAM.  The RCM3200 contains a Rabbit 3000 processor clocked at 44.2 MHz and has 512K flash memory and 768K of RAM.

The amount of free memory varies between controllers and is primarily a function of the application code.  As of this writing, the amount of free memory on the only RCM2200 controllers running DEBROS is approximately 38K.  The RCM3200 has an additional 256K or RAM, which allows considerable room for growth compared to the RCM2200 based controllers.  Also, the RCM3200 runs at twice the speed of the RCM2200, allowing it to do roughly twice as much work each second.

As of this writing, there are about 260 Rabbit-based controllers, 84 of which use the RCM2200.  All new development is done using the faster RCM3200.

As with the existing ModBus method, this plan covers only the implementation of a CA server for the embedded controllers, NOT a CA client.

An overview of the DEBROS platform

Before launching in to the details of the design for the CA server, a bit of background on the environment it has to work in to will help explain the mindset that went in to it and why I believe the proposed design is a good fit.

The primary task of all DEBROS controllers is to monitor and/or control a device or a group of devices.  To accomplish this, a single application-specific task runs at a higher priority than any other task and contains a “run-to-completion” loop that executes on a periodic basis (usually once every 10 or 20 ms).  Because it runs at a higher priority than any other task, it cannot be preempted.

Regardless of the actual application logic, the loop performs the following steps each time it executes:

- Update the state of the outputs (using the values computed at the end of the previous pass)

- Read the state of the inputs

- Interpret the meaning of the current state (using the current state and any changes in the state since the previous pass)

- Determine the state to set the outputs to during the next pass

It is important to note that inputs and outputs do not refer just to hardware signals, but to all the global variables that the application loop accesses.

In most cases the application task is responsible for real-time control of one or more devices and it must be able to execute on a regular, predictable schedule with no excessive delays.  Any processing that might cause it to block for an unpredictable or excessively long period of time (such as reading from or writing to socket connections) must be avoided.  Care must also be used to avoid disabling the scheduler or otherwise lock a resource used by the application task for any significant period of time.

Most every other process runs on a much less rigid schedule and performs processing tasks that are much less predictable in terms of delays and CPU load.  And while they handle a wide variety of tasks, most of them are, in one form or another, managing some form of communication.

Some tasks communicate with external clients over a socket (ModBus, fast-data, readWave).  Some communicate with an external device over a serial port (ovenCom on ECROven controllers), or with a display that is part of the controller (lcdUI on Modular PS controllers).  Some are initiated by users using a command-line shell (reg, watch, scope).  Yet others manage the configuration of the controller (backupd, configd).  Whatever their areas of responsibility, most tasks access a subset of the global variables, many of which are shared with the application task.

While the task performed by each of these processes is the same for each type of controller, the set of values used in many cases is dependent on the application.  To allow the code for these tasks to remain the same for all controllers, the application specific properties are stored in tables generated at compile time using macros and the C preprocessor.

The tables that drive these processes contain things like the names of the values, a pointer to the value, their data types, their size, and pointers to functions to call for any application-specific handling that needs to be done.  The use of macros makes setting the properties of each entry in these tables simple and makes them easy to read and maintain.

This table-driven approach works well for the generic processes common to all DEBROS controllers precisely because they DON’T know or care what the values represent or how they are related to other values.  For those cases where application-specific handling is required, the table provides a pointer to an application-specific function to handle it.

The application function accesses a large number of global variables each time through its loop, and it does this once every scheduler tick which is usually set to 50 or 100 times every second.  It is therefore critical that access to these values be as efficient as possible.  Requiring the use or wrapper functions or lookups via a table causes delays that are not acceptable.

The much lower rate and volume at which other tasks access these values coupled with the much less critical timing constraints means that the overhead added by use of the table-driven approach is a more than acceptable tradeoff for the ability to keep them generic.

While writing this document it occurred to me that issues I faced where similar to what the designers of EPICS had to deal with:  How to provide a standard structure and format for defining the device-specific data elements and yet not affect the efficiency of the code that uses it.  Like the Rabbit modules, the architecture that EPICS was originally designed for had limited resources.  And they needed something that manage to do the processing for hundreds or thousands of devices at once, not one!

Not surprisingly, the solution I came up with (even before I had conceived of DEBROS) is basically the same as that used in EPICS.

In EPICS, PVs are grouped in to records.  The set of values that make up a record are called fields, and the code that manages all the fields in a given record is the record logic.  A text file called a DataBase Description (.dbd) file provides the definitions for a record’s fields and a C code file provides the record logic.  The .dbd file is passed through a parser that generates C code that describes/defines the fields in the record.  The C code generated from the .dbd file includes a standard C variable definition for each field.  This C code is then included in the record code when compiling it.  An EPICS database can contain multiple instances of a record, each one having a unique name.  Each combination of the record name and a field name is a unique PV name that is available on the IOC hosting the record.

In DEBROS, the REG macros and the C preprocessor provide the functionality of the .dbd file and the application that parses it, and the application task provides the record logic.  Each controller is equivalent to one instance of an EPICS record, and each variable defined by a REG macro is one of the fields in the record.  The fields are currently exposed as ModBus registers using the DEBROS ModBus server (and other commands and interfaces), and the proposed CA server will expose the same set of fields as CA PVs.

In both cases, the end result is the same:

- The processing of the macros/.dbd file defines a standard C variable for each PV.  The application/record code is fast and efficient because it doesn’t have to call any functions or do anything special to access them.  And while the application/record code is running, it has exclusive access to them.

- The processing of the macros/.dbd file also creates the tables that provide the information other tasks (such as a CA or ModBus server) use to access the register/field/PV values.  This allows them to work without any application-specific changes.

Additional considerations for the existing ModBus implementation

While DEBROS provides many of the advantages and capabilities of a modern operating system like Linux, the fact remains that the Rabbit systems are 8-bit processors and have very limited CPU power and memory.  Any plan for changes or additions to the functionality must take these limitations in to account, not only in terms of the effect on the implementation of the communications protocol, but on the overall affect on the ability of the controller to perform its primary task of controlling and monitoring some device (or group of devices).

A good example of how these considerations can shape the design is the existing implementation of the ModBus service on the DEBROS platform.  The simplicity of the ModBus protocol itself makes it very tempting to represent the values as a contiguous array of 16-bit values in memory and simply read and write to these arrays in response to each ModBus packet.  This would indeed make for a very small, simple, and very efficient ModBus server.  But it would totally ignore all the other requirements of the application that has to use and maintain those values.  The following are just some of the things that such a simple, “efficient” approach would make difficult or impossible:

- Checking the values to make sure they are valid and within allowable ranges

- Triggering processing in response to changes in the values

- Handling dependencies between values

- Insuring values that span multiple registers are treated as a single value (and ensuring ModBus commands reference only complete values)

- Keeping the most frequent kinds of access to the values by other tasks (usually reads) simple and efficient

- Providing diagnostic interfaces for aiding in development and maintenance

- Support for multiple protocols from a single set of definitions

- Insuring portability to different hardware platform (e.g. the model must not break due to differences in data alignment or representation on different platforms)

- Simple, straightforward declaration of the meaning of the values, their format, and all the values that provide the information needed to achieve the above

While all these issues must be considered, avoiding the need for special functions or macros in the code that add overhead just to access the values was of primary concern.  Experience has shown that the values that need to be exposed to external clients are some of the most referenced values in the code.  If every reference to these values had to be done via some function call or a table lookup, the resulting code would be much larger, slower, and harder to read.

The approach used in DEBROS for defining and managing values that need to be accessible to external clients takes all these in to account.  The “efficient” method of an array of values would have made the ModBus interface very small and fast, but it would have been at the expense of everything else.  The method used in DEBROS turns this around:  Using just a single macro to define each value, it creates an array of the data structure that contains all the info needed to safely read from and write to the values for external clients, but keeps all the application code that has to access these values simple and efficient by also declaring a standard C variable for each value.  This translates in to far fewer instructions to access the values, keeping the majority of the code small and fast.

The advantages of this approach are even more apparent when you consider that the compiled code is not only much smaller and faster this way, but that the application-specific code and supporting services generally accesses these values far more frequently than the tasks servicing external clients.  In the case of the existing DEBROS controllers, the main application loop accesses them 50 or 100 times per second, while the ModBus service reads them maybe 5 to 10 times a second and writes to them even less frequently.  Add to this several other services and commands that do things like check these values for changes and the differences between the methods adds up really quickly.  Simple math will tell you the result is far less CPU cycles required overall than the method that ignores everything but communications with external clients.  And given that the data structure describing each value would be required in any case, it doesn’t use any more memory for the data, and it saves quite a bit on memory for the code.

The other key feature of this approach is that it was designed from the start with the expectation that the ModBus protocol would NOT be the only method used to access the values.  There are, in fact, multiple interfaces right now that make use of the data structure created by the existing macros:  In addition to the ModBus client, these are:

- The “reg” command:  A shell command for viewing and/or modifying one or more values at run time.

- The “watch” command:  A shell command that displays a continuously updated group of values.

- The “software scope” interface:  The ability to connect directly to a controller using the NSCL QtChart application and get a continuously updated plot of a selected group of values.  The current state of each value is copied to the data stream once each pass through the main application loop, so the resulting plot will include EVERY instance that the application task sees and processes for these values.  This is very helpful in debugging or monitoring control loops, and requires very little CPU time.

Several other interfaces (such as the tables that manage saving and restoring configuration values) also benefit from the fact that they can treat the values that are exposed to external clients in the same fashion as any other memory variable.

In summary, any design for implementation of a communications protocol must take in to consideration a much broader scope than just the efficiency of the communications interface itself.  The existing model is highly integrated into and provides critical features required by all the DEBROS-based applications.  It was designed from the start to minimize the resources needed overall and to be flexible enough to support protocols other than ModBus.

Adding support for the Channel Access protocol

Use of the CA protocol should provide several advantages over the current use of ModBus:

- Simpler integration in to the EPICS control system:  Since CA is the “native” protocol for communication between EPICS servers, no additional layer of software is required to communicate with CA capable devices.

- Faster, more frequent updates:  One of the main disadvantages of the ModBus protocol is that it is a polling protocol.  The current state of a group of values is sent only once for each request.  The only way to maintain an up to date copy of a group of values is to continuously send requests to the device.  CA supports an asynchronous publish/subscribe model where the client is automatically informed of changes to the data without having to ask.  This could significantly cut down on network “chatter” and improve response times compared to the ModBus model.

- Standardized support for strings and floating-point values:  While use of multiple ModBus register values to represent values that won’t fit in a single register is common practice, there are no standards for what kinds of values are universally supported or how they are formatted or processed.  For ModBus, this means (among other issues) that the client or the server may have to reorder the bytes for a given client/server combination.

- Ability to define and maintain a standard set of common values:  All ModBus values are referenced via a numerical “address” or index.  This makes it impractical to maintain a standard set of values common to all applications, since any additions will have to use the next unused indexes for each application, which will likely be different for each application.  The only alternative would be to define a block of “unused” values and hope that you never outgrow it. Even if you make it large enough to insure you don’t outgrow it, the unused values are a waste of resources and can significantly slow down the communications interface.  This is not an issue for the CA protocol because it refers to values by name.

Recommended approach for implementing CA on DEBROS

DEBROS supports much of the standard BSD socket library API as well as the stdio file descriptor based I/O method and function calls.  This means any solution written for DEBROS now should require very little work to port to a different hardware platform in the future.

As with all resources on such a small system, the maximum number of tasks and sockets in use at any one time is quite small and must be taken in to consideration in the overall design (see the end of this document for a summary of current limits and typical memory usage on DEBROS devices).

In CA, each data element is referred to as a PV (Process Variable).  UDP broadcast packets are used to discover which server provides a given PV.  Clients access PVs by opening a TCP connection (a CA Virtual Circuit) to the appropriate server and creating a Channel for each PV.  The PV for a Channel can be read from or written to on request, and Subscriptions (Event Monitors) can be created on Channels to cause the server to notify the client of various changes to the PV’s state (such as any significant changes in the value).

A single Virtual Circuit is normally used for all communication between a given server and client.  Multiple Channels can be created for a given Virtual Circuit, and multiple Subscriptions for each Channel.

The following is a typical sequence of events for accessing a set of PVs:

- Client sends out a UDP broadcast SEARCH packet for each PV it is interested in.

- For each SEARCH packet they receive, each CA server checks its list of PVs for a match.  If a match is found, the server responds using a UDP datagram packet.

- Using the first response received (if any) for each PV name, the client establishes a Virtual Circuit (a TCP connection) to the responding server (if it doesn’t already have one).

- The client uses the Virtual Circuit to create Channel connections for the PV’s managed by each server, and Subscriptions on each Channel for which it wants to automatically receive notifications of changes.

- The server periodically does the following:

  - Send out beacon packets

  - Check for and accept new connections

  - Process any messages received

  - Check for changes to the PVs that have at least one Subscription

  - Send messages for any of the detected changes

Overall design

The basic responsibilities of a CA server are as follows:

- Generate beacon packets

- Listen for and respond to CA SEARCH packets

- Listen for and establish new Virtual Circuit connections on request

- Process commands received on each Virtual Circuit

- Maintain lists of Virtual Circuits, Channels, and Subscriptions

- Periodically check for events that apply to each Subscription and send out messages as needed

The traditional and most straightforward way to handle all this would be to have the inetd process listen for packets and launch a new process to handle each new connection or broadcast packet.  But this approach has some significant drawbacks:

- Processing for a CA SEARCH packet won’t even begin until the inetd task detects that a packet was received.  Since it is responsible for listening for other services as well, this adds to the response time.  Given that such packets can be fairly high in volume, the delay could have a significant impact in both response time and how many packets get dropped.

- The time required to create a new process for each packet would likely cause even worse delays and packet loss.  The likely result would be significant delays in establishing the Channel connections for each PV.

- It requires using more of one of the many limited resources on the DEBROS platform:  The number of concurrent processes.

- It makes sharing information across Virtual Connections more difficult and less efficient.  Given that there will likely be considerable overlap between the Channels and Subscriptions across the Virtual Connections, this could have a large impact on performance and resource usage.

I believe the most efficient way to handle this (particularly on DEBROS) would be to have a single process that works as follows:

- Create a UDP socket to listen for CA SEARCH UDP broadcast packets

- Create a TCP socket to listen for requests for new Virtual Circuit connections

- Generate a UDP broadcast CA beacon packet periodically

- Accept new Virtual Circuit connections (when possible and within set limits)

- Do a blocking read with a short timeout on all open sockets (including the listening ones).  When the process resumes, process any packets received on each of the open sockets, check for events on PVs referenced by Subscriptions, and send event messages for each Subscription as needed.

The single-task approach also makes it much simpler to avoid redundant processing both across Virtual Circuits and even within the same Virtual Circuit.

The proposed data structures

To provide the functionality outlined in the CA Protocol Specification, a CA server needs to manage the following information:

- A list of Virtual Circuits

- A list of Channels for each Virtual Circuit

- A list of Subscriptions on each Channel

- The Client ID (CID) for each Channel

- The Server ID (SID) for each Channel

- The Subscription ID for each Subscription

The following are factors that I believe are relevant to the choice of data structures for implementing a CA server on DEBROS:

- Memory usage needs to be kept as low as possible, particularly if there is to be any chance of using the resulting implementation on the Rabbit modules that have only 512K of memory (which have less than 40K of free memory left for a CA server implementation).

- The largest number of data elements currently defined on any of the existing controllers that would be accessible via CA is 156.

- With only 2 exceptions that don’t apply to the DEBROS implementation, the only time the CA server has to send a CID value to the client is in response to a message where the client provided the CID value (based on testing done by Vasu, this is another place the Protocol Specification is wrong – his tests indicate that the CID IS returned in at least one instance where the specification says that the SID is returned – see discussion later on for details of my review of the Protocol Specification).

- The currently set upper limit for sockets in DEBROS is 12 TCP and 4 UDP, which includes sockets that are just listening for a new connection on a specific port.   Given that on average half of these are already in use by existing services (and that we need to keep a few available for diagnostic use), the CA server on DEBROS will need to limit the # of simultaneous CA connections to 2 or 3.

- The hardware DEBROS is currently running on consists of low-speed 8-bit microprocessor units (think of an extended Z80 architecture running at 22Mhz or 44 MHz)

- Some additions have been made to the existing macros for defining the values currently used by the ModBus interface to support the needs of the CA interface.  Some functions (described below) have also been written that provide a CA/PV compatible interface to those values.

Given the circumstances described above, I believe the following data structures will provide the information needed by the DEBROS CA server in a form that will allow it to run efficiently and with minimal memory requirements.

NOTE:  The methodology described here will support all the event CA types.  However, to minimize the memory requirements and CPU overhead, we will likely support only VALUE change events.

Structures to keep track of the information for each Virtual Circuit:

//--- Info about a CA Subscription ---

typedef struct  {

uint srvID;     // SID for the subscription (index in to CA_Con.chans[])

ulong subID;    // SubscriptionID provided by the client

uchar monMask;  // monitor mask provided by the client

uchar dbrType;  // format for asynchronous event messages

} CA_Subscrip;

//--- Info about a CA Channel ---

typedef struct  {

uint  pid;      // PV ID value returned by pvGetPID()

ulong  cid;     // client value provided by the client

} CA_Chan;

//--- Info about a CA connection (Virtual Circuit) ---

typedef struct  {

fd_t  vcfd;                      // fd for connection; <0 means unused entry

bool  subEnabled;                // Subscriptions enabled/disabled flag

CA_Chan chans[CA_MAX_CHANS];     // list of open Channels: SID=index into this array

CA_Subscrip  sub[CA_MAX_SUBSCR]; // list of Subscriptions

char username[USER_NAME_LEN];    // user name provided by client

char hostname[HOST_NAME_LEN];    // host name provided by client

} CA_Con;

CA_Con far caCon[MAX_CA_CONS];

How these data structures support the CA server functionality:

- caCon[..]:  The list of Virtual Circuits.  MAX_CA_CONS sets the upper limit on the number of Virtual Circuits that can be opened at once.

- caCon[..].vcfd:  The file descriptor for an open Virtual Circuit.  A negative value indicates a caCon[] entry that is not in use.

- caCon[..].subEnabled:  A flag indicating whether or not Subscriptions are enabled or disabled for the Virtual Circuit

- caCon[..].userName[..]:  The user name associated with the Virtual Circuit.  Will have the value “unknown” unless the client provides one using a CLIENT_NAME message.  Kept strictly for diagnostic reasons at this time.

- caCon[..].hostName[..]:  The host name associated with the Virtual Circuit.  Will have the value “unknown” unless the client provides one using a HOST_NAME message.  Kept strictly for diagnostic reasons at this time.

- caCon[..].chans[..]:  The list of Channels for a Virtual Circuit.  Each element represents one of the possible Server channel ID (SID) values (i.e. the index in to this array is the SID for the Channel).  CA_MAX_CHANS sets the upper limit on the number of Channels that can be opened on a single Virtual Circuit.

- caCon[..].chans[..].pid:   The value returned by pvGetPID().  A value of 0xFFFF indicates an unused SID (i.e. an unused entry in the caCon[..].chans[..] array), while all other values represent one that is in use.

- caCon[..].chans[..].cid:  Contrary to what the Protocol Specification says, Vasu’s work to date indicates that the Client channel ID (CID) DOES need to be returned in some messages that are not an immediate response to one that contained the CID, so it appears we WILL need to store the CID value for each Channel.

- caCon[..].sub[..]:  The list of open Subscriptions for a Virtual Circuit.

- caCon[..].sub[..].srvID[..]:  The SID value for the subscription, which is an index in to CA_Con.chans[..].  A value of 0xFFFF indicates an unused entry in the CA_Con.sub[..] array.

- caCon[..].sub[..].subID:  The SubscriptionID provided by the client in an EVENT_ADD message.

- caCon[..].sub[..].monMask: The Monitor Mask bits set by the client (using one or more EVENT_ADD messages for a given SubscriptionID).  CA_MAX_SUBSCR sets the upper limit on the number of Subscriptions that can be opened on a single Virtual Circuit.

The following functions were added to provide the logic necessary to support a CA/PV service:

int pvGetPID(char *pvName);

int pvGetInfo(int pid, PV_INFO *pvInfo);

int pvGetVal(int pid, uint toType, uint toSize, void *toVal, uint event);

int pvSetVal(int pid, uint fromType, uint fromSize, void *fromVal);

int pvCheckEvents(int pid, uint checkEvents);

void pvCheckAllEvents(time_t *dateTime, uint *ms);

These should be used as follows:

- pvGetPID() returns an ID for a PV given the PV’s name (minus the hostname or record name portion), or -1 if there is no PV with the specified name.  This function is used to process SEARCH and CREATE_CHAN messages.

- pvGetInfo() returns information about a PV (currently includes the PV’s name, dataType, size, and a flags value that includes bits for read and write access), or -1 if there is an error.  Used primarily for processing CREATE_CHAN messages.

- pvGetVal() retrieves either the current value for a PV or the value it last had for the specified event.  Returns 0 on success or -1 if there is an error.  Used for EVENT_ADD and READ_NOTIFY messages.

- pvSetVal() changes the current value of  a PV.  Returns 0 on success, or -1 if an error.  Used to process WRITE commands.

- Each call to pvCheckEvents() does 2 things:  It returns a bit-mask of any events that the last call to pvCheckAllEvents() detected for a PV, and adds to the events to be checked for the PV the NEXT time pvCheckAllEvents() is called.  This is used to process EVENT_ADD messages and to determine which subscriptions a new READ_NOTIFY message should be generated for.

- pvCheckAllEvents() is called once to update the bitmask of events that happened for each PV since the last call to pvCheckAllEvents().  It will check ONLY for the events specified in calls to pvCheckEvents() since the LAST time pvCheckAllEvents() was called.  This works in tandem with pvCheckEvents() to support the generation of READ_NOTIFY messages.  The dateTime value it returns is the value that should be used in any READ_NOTIFY messages that include a timestamp for the value sent in the message.

The pvCheckEvents() and pvCheckAllEvents() must be used together as follows:

- pvCheckEvents() is called for each successful EVENT_ADD message.  This insures the specified events will be checked for during the next call to pvCheckAllEvents().

- To generate a new set of READ_NOTIFY messages, call pvCheckAllEvents() once and then pvCheckEvents() once for each subscription.

This approach keeps the interface and the task of keeping the list of events to be checked for each PV very simple.  The basic idea is that pvCheckAllEvents() keeps 2 event bit-masks for each PV:  The events to be checked next time it is called (checkEvents), and which ones occurred when it was last called (eventsOccured).  Each time it is called it does the following for each PV:

- Clear eventsOccured mask

- Update the eventsOccured mask based on the checkEvents mask

- Clear the checkEvents mask

After a call to pvCheckAllEvents(), pvCheckEvents() is called for each subscription, which does 2 things:

- It tells us which events occurred for the PV

- It adds to the events to be checked the next time pvCheckAllEvents() is called

This approach insures that the checkEvents mask for each PV always matches the current list of subscriptions each time pvCheckAllEvents() is called without the need for a bunch of counters to keep track of when subscriptions and their events are added or deleted.

Limitations:

The proposed implementation allows the following scenario to occur:

- A client creates a channel and then a subscription on the channel

- The value of the PV for the subscription changes

- The client reads the current value of the channel

- The server does its periodic test of all channels with subscriptions and finds that the value changed since the last time it checked so it sends event messages to all clients with a subscription, including the one that already got the most recent value via an explicit read request.

I don’t believe the CA protocol document specifies what SHOULD be done in such cases.  It is not a likely case, but it would be perfectly reasonable to expect that, if a client already got the most recent value via an explicit read request for a channel that it will not receive an additional (asynchronously sent) event message with the same value sent in response to the most recent read.  And even if it does occur, the worst that should happen is some redundant processing by the client and the server.

A word about CID values

If you are familiar with the CA protocol, you may have noted what looks like a major deficiency in these data structures:  It does not provide for saving the CID value the client sends in some of the messages!

The reason for this is as follows:

(NOTE:  Work done by Vasu after this was first written indicates that the protocol specification is incorrect and that CID values probably DO need to be stored. The proposed data structures and the memory requirements have been updated to include the CID value, but this discussion is left here as a reminder to verify the need for keeping track of the CID value).

- The CID values are 32-bits (4 bytes) each.  Multiplying that by CA_MAX_CHANS * MAX_CA_CONS means a LOT more bytes of memory.

- There are only 2 message types where a CA server would ever need to send a CID value that is not in immediate Response to a Request just received from the client:  ACCESS_RIGHTS and SERVER_DISCONN.  But neither of these cases applies to the DEBROS implementation:

  - The ACCESS_RIGHTS message needs to be sent in response to a successful CREATE_CHAN Request.  But the rights for a Channel will never change in the DEBROS implementation, so it will never have to send one at any other time.

  - The SERVER_DISCONN message is sent when the “… server disconnected the channel”.  About the only time this might be useful in the DEBROS implementation would be to send one of these messages for every channel when it is doing a controlled shutdown.  But DEBROS already closes all socket connections before doing a controlled shutdown, which certainly should cause the client to realize that the connection is no longer valid without us first having to tell it that each individual Channel is being disconnected.  And given that the likely response of a client would be to try to create the channel again using the existing Virtual Circuit, this use of this message would seem to be somewhat questionable, and possibly counter-productive (i.e. the client might keep trying to recreate the channels while DEBROS is trying to close them all!)

As for keeping track of the pairing of CID and SID values, there is only one Request type that sends both the CID and SID values, and an incorrect pair of values would have no adverse affect on the server.  So nearly doubling the memory needed just to check for a buggy client message in one case seems unwarranted.

Summary of memory requirements for proposed method

Bytes needed per Subscription (BPS):  8		Example:

CA_MAX_SUBSCR    =  150

Bytes needed per Channel (BPC):  6				CA_MAX_CHANS      =  150

USER_NAME_LEN     =   16

NUM_PVS_DEFINED =  150

Bytes needed per Virtual Circuit (BVC): 		Example:

(BPS * CA_MAX_SUBSCR)			   8 * 150

+ (BPC * CA_MAX_CHANS) 			+ 6 * 150

+ 3 (for vcfd and subEnabled)			+ 3

+ USER_NAME_LEN 				+ 16

+ HOST_NAME_LEN			           	+ 16              .

2135  bytes

Bytes per PV per supported event type (BPE):

2   (2 masks for up to 8 event types)

+ 4   (per event, for value pointers)

+ 6   (per event, avg # of bytes for data) *

= 12  (for 1 event, avg 3 bytes data size)

(independent of # circuits, channels, subscriptions)

BVC  +  BPE * NUM_PVS_DEFINED:			+ 12 * 150    .

Total:	   3935  bytes

+ code size (???)

(*)  Current, last, and minimum change values, assuming an average value size of 3 bytes

How the data structures would be used

The CA server process will have a loop that executes frequently (about least 10 times per second) and performs the following tasks each time (the ms values given are a guess at a good starting point – we may need to tweak them to achieve a good balance between response time and CPU load):

- If time since we last started the loop is < 50ms, then sleep for the difference.

- Check the current number of connections.

  - If the max has been reached, and we have a listening socket, close it.

  - If the max has not been reached, and we do not have a listening socket, create one.

  - If we have a listening socket, check to see if there is a new connection request.

  - If we have a new connection request, initialize the first unused caCon[] entry for the new connection.

- Check for a UDP broadcast packet.  If one was received, process any CA messages it contained.

- If it has been a while since we sent a beacon packet (15 or 30 seconds?), send one now

- If no Virtual Circuits open, repeat the above.

- Do a blocking read with a timeout of about 20 ms using poll() on all the open Virtual Circuits.  Process any CA messages received from each of the connections as follows:

  - Read from a PV:  use pvGetVal() to get the value in the specified format

  - Write to a PV:  use pvSetVal() to change the value of the PV

  - New Channel:  Add PV to the list of Channels

  - New Subscription:  Add it to the list of Subscriptions and call pvCheckEvents() to update the checkEvents mask for the PV

- Call pvCheckAllEvents() to update the eventsOccured mask for each PV that has at least one subscription

- Call pvCheckEvents() for each Subscription to get the eventsOccured mask for the subscription and to update the checkEvents mask that pvCheckAllEvents() will use for the PV the next time it is called.  Send a READ_NOTIFY message for each event that occurred.

Some misc comments on other aspects of implementing CA on DEBROS:

Data Types

The macros currently used to define values accessible via the ModBus interface include support for 5 of the 10 types that CA supports.  Support for all but the DOUBLE type could be easily added.

Access Rights

Access rights based on the client and host names is probably beyond the scope of what is practical to support.  Most likely we will just report access rights as Read Only or Read/Write based on the definition of the data elements, which already support these settings.  However, we should at least store the client/user and hostname information sent by the client for each virtual circuit as this could be useful for diagnostics and monitoring.

Performance issues

Support for asynchronous events and data conversion are probably the most significant differences between ModBus and CA.  The ModBus server is entirely passive.  It sits idle until it receives a query.  Supporting CA Channel Events and data conversions will likely require far more CPU cycles for the CA daemon than the ModBus daemon currently uses.

The number of CPU cycles used by the CA daemon over a period of time will vary based on the following:

- The total number of open Channels and Subscriptions

- The number and types of Subscriptions on each Channel

- The total number of PVs referenced by all the Channels

- The size and type of data represented by each PV

- How frequently the monitors for the Subscriptions are checked

- How many Subscriptions specify a data type that is different than the one the data is stored in

Note that the CA daemon doesn’t really care how many Virtual Circuits (i.e. clients) the Channels are spread across.  It is the number of Channels and Subscriptions (and how unique they are) that determines the load.  Of course, the number of different clients IS an issue in terms of using up more of the very limited number of socket connections.  But it should have little effect on the load experienced by the CA daemon.

An Analysis by CA message type

Once again, the logic for a CA server in a nutshell:

- CA servers host data elements called Process Variables (PVs)

- CA clients form connections to CA servers to get access to PVs

- Clients send broadcast messages to discover which servers host the PVs they are interested in

- Clients use 3 types or layers of connections to communicate with a server:

  - A Virtual Circuit (normally a TCP connection) is used to send and receive CA messages between a given client and server

  - A Channel is tied to a specific Virtual Circuit and represents a logical connection between a client and one of the PVs on a server

  - A Subscription is tied to a specific Channel and is used to specify the type of event messages the client wishes to receive asynchronously for the Channel’s PV

Judging by some of the outdated values in the definition of the message used to create Subscriptions, earlier versions of CA allowed each client to specify values used to determine when certain events occurred (e.g. how much must a PV value change to trigger a VALUE change event).  Since these values are now ignored, that approach was most likely found to add too much overhead to the server.

Currently the values used to determine when an event has occurred for a given PV is the same for all clients and subscriptions.  This means the work required to determine when an event has occurred is independent of the number of subscriptions or the number of clients.

In contrast, if each subscription could specify its own set of values for defining when an event occurred, then the amount of work would (potentially) be multiplied by the number of subscriptions.  And the number of messages generated for each subscription would be largely dependent on the values specified.  For example:  If one subscription used a range that resulted in a 0 or near 0 deadband for value change events while others used a larger range, the one with the small range could cause it to get a message for every single change, no matter how small.

The following provides a fairly detailed analysis of how each of the CA message types could be handled by the proposed design for the DEBROS CA server.  This analysis is based on the protocol as described in the Channel Access Protocol Specification dated 2004-05-31.  In many cases, the Protocol Specification is incomplete and/or inconsistent.  My analysis includes a discussion of each such case and my assumptions about what constitutes correct behavior.  Some further study of the behavior of existing clients and servers using a protocol analyzer will be required to confirm or deny these assumptions.

NOTES:

- In EPICS (the control system software the CA protocol was designed for), PVs have names of the form:  RECORD.FIELD, where RECORD refers to a specific instance of a predefined group of fields, and the set of FIELD names is the same for every instance of a given record type.  For the DEBROS implementation, we should use the controller’s hostname for the RECORD portion of the PV name to distinguish them from all the other controllers of the same type.

- Most records provide a minimum set of fields that all record types should supply.  One of these is the RTYP field, which contains a name indicating the type of record and so can be used to infer what other fields you can expect to find in the record.  For the DEBROS implementation, we should make sure that RTYP is one of the PVs defined for every controller type and use the ApplAbbrev[] string for its value.

- The CA server task must use proper locking when modifying any global data that may be accessed by other tasks.  For VERY short pieces of code (e.g. writing to or reading from a single global value), a critical section can be used.  For anything larger, a semaphore or the scheduler lock should be used.  But care must still be taken to keep such locks very short and of predictable length (i.e. no calls that can block, unlock/relock if necessary to keep the lock interval short, etc).

- While most CA Request messages have a defined Response message, communication with a CA client is not always a simple receive a request, send the defined response.  In the case of a SEARCH Request, a response may or may not be sent.  In several cases, a different message (usually but not always an ERROR message) may be sent INSTEAD of the defined Response message.  In the case of a successful CREATE_CHAN Request, 2 different Response messages are sent.  See the detailed discussion of each message type that follows for details and assumptions where the specification is not clear.

- The odd grammar in many of the comments, coupled with the fact many of the statements are contradictory and/or don’t make sense as is (but do if they are modified a bit), leads me to believe that English was not the primary language of whoever wrote them.  This is most unfortunate (and frustrating) given the technical nature of the document, since having unclear and contradictory information makes it difficult to implement a server correctly.

- Programing tip:  While this doesn’t have anything to do with design of a DEBROS CA server, the following programming tip is something I offer to help insure as efficient an implementation as possible.

I would have thought the following would be fairly obvious to anyone that has looked at the code generated by the Softools compiler.  But I found that even the original code for the flash file system from SHDesign’s suffered from this problem.  And this is a guy who presumably has spent many more hours than me doing esoteric things with the Rabbit modules.  I certainly found his sample code for various things very helpful.  But maybe he never had a class in compilers.  In any case, may the reader forgive me if this IS already obvious to them…

Use pointers rather than subscripts!

Use of a subscript generally requires multiplication.  For a piece of code that has just a single reference to an element of an array (and that reference is not inside a loop), then using a subscript is probably faster than setting a pointer and then using the pointer.

But if the array element is a structure, and the code references multiple elements of the structure, then setting a pointer and using the pointer to access each of the elements is generally significantly more efficient.  And if the reference is in a loop that iterates through multiple elements of the array, initializing a pointer outside the loop and incrementing it within the loop is generally much more efficient, because the compiler can just add a fixed constant to the pointer each time through the loop, rather than having to multiply the index by some constant (and, if accessing an element in a structure, having to add a constant value anyway).

Optimizing compilers can help reduce or (in the simplest cases) eliminate the extra overhead, but even then a sufficient number of general purpose registers is needed in order to perform such optimization effectively.  This is difficult enough on any platform, but the Rabbits are just extended versions of the original Z80 8-bit processor running at higher speeds.  Multiplies require many CPU cycles, and Rabbit processors don’t have much in the way of general purpose registers.  Add to that the fact that the data structures we will be using will be in extended memory, and the cost of each piece of code using subscripts rather than a pointer adds up really quick (both in terms of CPU time used and the size of the code).

As I said, I was amazed to see the SHDesign’s code full of subscripts.  There simply isn’t any excuse for this when multiple lines of code are referencing the same structure, or an array of some structure.

For example, code structured like this:

con = caCon + vc;  pSub = con->sub;

for (i=0; i<CA_MAX_SUBSCR; ++i,++pSub)  {

if (pSub->srvID != srvID) continue;

if (pSub->monMask) …

}

is almost always smaller and faster than:

for (i=0; i<CA_MAX_SUBSCR; ++i)  {

if (caCon[vc].sub[i].srvID != srvID) continue;

if (caCon[vc].sub[i].monMask) …

}

NOTE:  The use of the subscripted format in this document when referring to the suggested data structures is for clarity only.  It is NOT meant to suggest how references to the data structures should be written!

So, finally we get to the analysis for each message type:

4.0  CA_PROTO_VERSION

Sent to the client when the CA server task gets a new TCP connection.

The CA server task will normally maintain a TCP socket in listen mode for receiving new connections (CA Virtual Circuits).  Each time through its loop it will check to see if a new connection is established.  If one is established, the server performs the following:

- Scan the list of Virtual Circuits.  For each one that is open, check to see if the IP address matches the remote address for the new connection.  If a match is found or all connections are in use, send an ERROR message with code ECA_MAXIOC.

- Mark the first free entry as used

- Save the file descriptor for the new socket connection

- Set the list of Channels to an empty list

- Send a CA_PROTO_VERISON message

4.6  CA_PROTO_SEARCH

For a CA client to establish a connection (a Channel) for a given data element (a PV, or Process Variable), it must know what CA server hosts it.  CA clients discover this by sending SEARCH Request messages using UDP datagrams (usually as broadcast packets).

Each message contains the name of one of the PVs the client needs to connect to.  Each time a CA server receives a SEARCH message, it needs to check to see if the PV name in the message matches any of the values it hosts.  If a match if found, the server sends a response which includes the TCP port number to use to create a Virtual Circuit connection, and the Channel ID (CID) provided by the client.

In DEBROS, the values accessible to external clients are those defined by the REG3 and REG4 macros.  The REG3 macro is used to define a read-only value, while the REG4 macro (usually) defines a read/write value.  These macros are used to define 2 arrays of RegInfo values, a structure that provides all the information needed to access each data element defined by one of these macros.  To support the CA server, some additions (as outlined earlier) will be made to this structure, as well as some changes to how they are used in the existing code (to simplify their use by the CA server and reduce memory usage a bit).

One of the values included in the RegInfo structure is the name of the global variable created by the macro.  Combining this name with the hostname for the controller provides a unique PV name for each value (e.g. HostName.VAR_NAME).

Unfortunately the description of the SEARCH message in the Protocol Specification contains several confusing statements and some errors:

- The Description for the message type says “Sent over UDP or TCP”, but the Comments for the Request and the Response say they are Sent/Received “as a UDP datagram”.

So is TCP supported or not?  If UDP is used for both the request and the response, when would you ever use TCP?

Also relevant to this question is this statement from section 1.3 Overall Operation:  “Client will use UDP to search for hosts and PVs, …”.

Given the number of statements stating or implying that UDP is how SEARCH messages are sent (see additional discussion below), and only one claiming TCP is an option, I believe that the one statement is in error and that UDP is, in fact, the only option.

- The Comments for the Request states “Sent as a UDP datagram” followed by “It is illegal to specify DO_REPLY flag whenever the message is sending as UDP datagram, regardless of whether broadcast or multicast is used”.

Stating that UDP is the only way to send a SEARCH Request and then saying that one of the two options for the Reply flag is illegal if UDP is used doesn’t make any sense, so I am assuming that there is something wrong with or missing from this statement.  Also, why mention broadcast and mulicast but not unicast unless unicast is treated differently?

The Comments for the Request also states: “Reply flag will be generally DONT_REPLY when searching using broadcast and DO_REPLY when searching using unicast.  When DO_REPLY is set, server will send a CA_PROTO_NOT_FOUND message indicating it does not have the requested channel”.

In combination with the other two statements, the most reasonable explanation that would eliminate the contradictions would be to assume the first statement about the DO_REPLY value should read:  “It is illegal to specify DO_REPLY flag whenever the message is sent as UDP broadcast or UDP multicast”.

- The table showing the contents of the Response Header AND the Comments for it both state that the CID value sent in the Request from the client should be returned in the Response.  But the Comments for the Response ALSO states: “In CA_V411 the server’s IP address is optionally returned in the CID field of the header allowing a CA server to act as a directory service.  If not, then the CID field is set to 0xffffffff”.

After spending more hours than I care to admit to trying to make sense out of these incompatible statements, it finally occurred to me that this is most likely just another error in the document.  While I eventually came up with some (rather convoluted) scenarios that would allow the CID field to be used as described for CA_V411, a much more likely explanation is that they meant to say that the SID field could be used this way.  That would avoid the numerous complications I was trying to puzzle out regarding the conflicting statements about what went in the CID field.  If they had simply tacked the statement on to the end of the one about the SID field - rather than making it a separate statement – I probably would have caught on a lot sooner!

Assuming my conclusions are correct, this is how a SEARCH message should be processed:

- If the PV’s record name (the part up to the first period) matches the controller’s host name (including case), then scan the list of PVs (the RegInfo array) for a variable whose name matches the rest of the PV name (ignoring case).

- If a match for both parts, then send a SEARCH Response packet.

- If the message was received in a unicast packet and the Replay flag was set to DO_REPLY, then send a NOT_FOUND message.

NOTE:  The reason case must be ignored for the variable name portion is to remain compatible with EPICS, which allows upper and lower case for the record name portion of a PV name, but forces all uppercase for the field name portion.

4.14  CA_PROTO_NOT_FOUND

This message is sent in response to a SEARCH Request in place of a SEARCH Response message when the following are true:

- The SEARCH Request was received in a UDP unicast packet

- The Reply flag in the request was set to DO_REPLY

- The requested name does not match any of the PVs hosted by the server

See the description in this document for handling of the CA_PROTO_SEACH message for more details.

4.23  CA_PROTO_ECHO

Simply an empty message that can be used to verify that a connection is still working.  Just send back the same empty message in response.

5.13  CA_PROTO_RSV_IS_UP

Message sent when the server task first starts and periodically while it is running.

The port number and IP address should be filled in, and the Beacon ID incremented each time another beacon is sent (starting with a value of 0 when the server task first starts).

5.17  CA_REPEATER_CONFIRM

Used on platforms where multiple CA clients on the same machine all need to process the CA packets received via UDP broadcast.  It does not apply to the DEBROS implementation.

5.24  CA_REPEATER_REGISTER

Used only on platforms where multiple CA clients need to process CA packets received via UDP broadcast.  It does not apply to the DEBROS implementation.

6.1  CA_PROTO_EVENT_ADD

Once a client has a Virtual Circuit open to the CA server task, and has created Channels (using CREATE_CHAN messages), it can add Subscriptions to its Channels that cause the server to send messages when one or more of the specified events occurs.

The type of event(s) to add to a new or existing Subscription is specified by a bit-mapped value called the Monitor Mask.  The following values are currently defined for this mask:

- DBE_VALUE

The difference in the current PV value and the last reported value is larger than some specified value (called the “deadband”).  While a deadband value of 0 can be used to send messages whenever a change in the value is detected, this should not normally be used for floating point numeric values that change frequently.

- DBE_LOG:

Just like DBE_VALUE except that it uses a different deadband value (usually larger than the one used for DBE_VALUE messages).

- DBE_ALARM:

The alarm value for the channel changed.

A request for a new Subscription is handled as follows:

- Check the value of the monitor mask in the Request.  If it is not valid, then send an ERROR message with code ECA_BADMASK.

- Locate the specified Channel in the list of Channels for the Virtual Circuit (caCon[vc].chans[..]).  If there is no matching entry, send an ERROR message with code ECA_BADCHID.

- Scan the list of Subscriptions (caCon[vc].sub[..]) looking for two things:  An existing Subscription with the specified SubscriptionID and the first unused entry.   If there is no matching entry, and no unused entry, send an ERROR message with code ECA_ADDFAIL.

- If there is a matching entry, update its monitorMask ((caCon[vc].sub[..].monMask |= requested mask).  Otherwise, create a new entry with the SubscrptionID and monitor mask provided by the client and add the entry to the list of Subscriptions for the Channel.

- Update the PV’s check-events mask using the Subscription’s new/updated monitor mask (i.e. PV’s checkEvents |= subscription’s monitorMask)

6.2  CA_PROTO_EVENT_CANCEL

Clears all event Subscriptions created by EVENT_ADD messages for the specifiedChannel.

An EVENT_CANCEL message is handled as follows:

- Scan the list of Subscriptions for the Virtual Circuit looking for a matching SubscriptionID (caCons[vc].sub[..].subID).  If no match is found, then send an ERROR message with code ECA_BADMONID.

- Delete the Subscription by clearing the subscription information (srvID = 0xFFFF, monMask = 0, subID = 0xFFFF)

- Send a EVENT_CANCEL Response message (which uses the Command code for an EVENT_ADD message!)

NOTES:

- We must NOT clear any of the bits in the referenced PV’s checkEvents bits because there may be other subscriptions for those events.  We COULD rebuild the checkEvents bits for the PV at this point, but the overhead in doing so is probably more than just letting it do the event check during the next pass. And if it turns there are other subscriptions for the same events, then rebuilding the mask now would just be a waste of time because it wouldn’t have changed.

- The response sent back to the client uses the EVENT_ADD command ID, NOT the EVENT_CANCEL command ID.  This is very odd, especially considering that the use of the fields in the header is different than their use in a true EVENT_ADD Response message.

6.4  CA_PROTO_WRITE

Requests a PV’s value be changed to the one provided in the WRITE message.

Note that this message differs from the WRITE_NOTIFY message in that the protocol specification states that there is no response to it.

Given that there IS a separate WRITE_NOTIFY (which DOES return a response), and that dealing with a response that may or may NOT arrive is rather problematic (see the discussion for the ERROR message for details), my assumption is that “no response” means not even an ERROR message is sent, regardless of what problems may occur while processing a WRITE message.

A WRITE message is handled as follows:

- Make sure the specified SID refers to an open Channel ((SID < CA_MAX_CHANS) AND (caCon[vc].chans[SID] != 0xFF)).  If not, do nothing.

- If conversion to the PV’s data type from the one specified in the Request is not possible, then do nothing

- Update the value of the PV converting the value in the Request as needed.

6.8  CA_PROTO_EVENTS_OFF

Used to disable sending of any updates for subscriptions on a specific Virtual Circuit (i.e. stop the server from sending EVENT_ADD messages on the Virtual Circuit).

An EVENTS_OFF message is handled by clearing the flag that enables the sending of subscription updates for the Virtual Circuit this message was received on (caCons[vc].subEnabled = false).

6.9  CA_PROTO_EVENTS_ON

Used to enable sending of any updates for subscriptions on a specific Virtual Circuit (i.e. allow the server to resume sending EVENT_ADD messages on the Virtual Circuit).

An EVENTS_ON message is handled by setting the flag that enables the sending of subscription updates for the Virtual Circuit this message was received on (caCons[vc].subEnabled = true).

6.11  CA_PROTO_ERROR

Used to inform the client of a problem with a request that failed but does not include an error code in the response.

An ERROR message is normally sent in place of the normal response when a Request that does not provide for a negative response has failed.

Refer to the discussion for an individual Request type for more details.

6.12  CA_PROTO_CLEAR_CHANNEL

Clears a Channel.  Any resources associated with the Channel are released and any future references to the channel are rejected.

A CLEAR_CHANNEL message is handled as follows:

- Make sure the specified SID refers to an open Channel ((SID < CA_MAX_CHANS) AND (caCon[vc].chans[SID] != 0xFF)).  If not, send an ERROR message with code ECA_BADCHID.

- Scan the list of Subscriptions (caCon[vc].sub[..]) looking for ones that have the specified SID (caCon[vc].sub[..].srvID == SID).  For each match, clear the subscription information (srvID = 0xFFFF, monMask = 0, subID = 0xFFFF)

- Clear the Channel entry for reuse (caCon[vc].chans[SID] = 0xFF)

- Send a CLEAR_CHANNEL Response message.

6.15  CA_PROTO_READ_NOTIFY

Returns the current value of the specified PV.

A READ_NOTIFY message is handled as follows:

- Make sure the specified SID refers to an open Channel ((SID < CA_MAX_CHANS) AND (caCon[vc].chans[SID] != 0xFF)).  If not, send an ERROR message with code ECA_BADCHID.

- If conversion from the PV’s data type to the one specified in the Request is not possible, then send an ERROR message with code ECA_NOCONVERT.

- Send a READ_NOTIFY Response message containing the value of the specified PV in the format specified in the Request.

6.18  CA_PROTO_CREATE_CHAN

Requests the creation of a Channel (a connection to the specified PV).

A CREATE_CHAN message is handled as follows:

- If the specified PV name does not start with the hostname of the controller followed by a period, return an ERROR message with code ECA_CHIDNOTFND.

- Scan the list of PV’s looking for one whose name matches the portion of the PV name following the period (ignoring case).  If no match is found, return an ERROR message with code ECA_CHIDNOTFND.

- Scan the list of Channels for the Virtual Circuit looking for an unused entry (caCon[vc].chans[..] = 0xFF).  If all the entries are in use, return an ERROR message with code ECA_ALLOCEM.

- Set the value of the new Channel entry to the index of the PV’s RegInfo structure (caCon[vc].chans[..] = index).

- Return a CREATE_CHAN Response using the index of the new Channel entry (the xx in caCon[vc].chans[xx]) for the SID value.

- Send an ACCESS_RIGHTS message with the CID value from the Request message and ACCESS_RIGHT_READ (for read-only values) or ACCESS_RIGHT_READ | ACCESS_RIGHT_WRITE (for read/write values).  Keep in mind that values defined with REG4 MAY have the read/write flag set to read-only (although this is unusual).

6.19  CA_PROTO_WRITE_NOTIFY

Requests a PV’s value be changed to the one provided in the WRITE message.

Note that this message differs from the WRITE message in that it returns a response.

A WRITE_NOTIFY message is handled as follows:

- Make sure the specified SID refers to an open Channel ((SID < CA_MAX_CHANS) AND (caCon[vc].chans[SID] != 0xFF)).  If not, send an ERROR message with code ECA_BADCHID.

- If conversion to the PV’s data type from the one specified in the Request is not possible, then send an ERROR message with code ECA_NOCONVERT.

- Update the value of the PV converting the value in the Request as needed.

- Send a WRITE_NOTIFY Response message indicating the WRITE completed.

6.20  CA_PROTO_CLIENT_NAME

Provides the user name associated with the Virtual Connection on the client side.

This currently has no affect on access rights on DEBROS and is kept purely for diagnostic reasons.

A CLIENT_NAME message is handled by copying the string supplied in the Request to caCon[vc].userName[..].  If necessary, the value is truncated to USER_NAME_LEN – 1 characters.

No response is sent for this message.

6.21  CA_PROTO_HOST_NAME

Provides the host name associated with the Virtual Connection on the client side.

This currently has no affect on access rights on DEBROS and is kept purely for diagnostic reasons.

A HOST_NAME message is handled by copying the string supplied in the Request to caCon[vc].hostName[..].  If necessary, the value is truncated to HOST_NAME_LEN – 1 characters.

No response is sent for this message.

6.22  CA_PROTO_ACCESS_RIGHTS

Returns the access rights for the specified PV.

On the DEBROS platform, access rights never change and are currently limited to READ or READ and WRITE.

Therefore, the only time this message is sent is in response to a successful CREATE_CHAN Request, at which point we still have the CID value from the Request.

6.26  CA_PROTO_CREATE_CH_FAIL

Indicates that a CREATE_CHAN Request failed.

The only time this message is sent is in response to a failed CREAT_CHAN Request, at which point we still have the CID value from the Request.

6.27  CA_PROTO_SERVER_DISCONN

Sent to the client to notify it that the server has disconnected a Channel.

Of all the messages a CA server might send, this is the only one that might be useful for the DEBROS implementation that requires the server to send a message with a CID value that it could not have gotten from the last Request sent by the client.

However, the only possible use for this in the DEBROS implementation would be in preparation for a controlled shutdown of either the server task or the OS.  Since the socket for the Virtual Connection that the Channels use would also be shutdown cleanly in these cases, use of this message by the DEBROS implementation seems redundant.  And since the likely response by the client would be to try to recreate the Channels on the connection that is about to be closed, it would seem to be a counterproductive use at that.

Most important from the standpoint of DEBROS is that, by NOT trying to use this one (questionable) case, we can cut the memory requirements nearly in half.

Overall conclusions

More memory and CPU will be required.  Support for Subscriptions and data conversion will be the most significant factors in increased CPU usage, and the need to allocate space for 2 copies plus a deadband value for most PV values will be the most significant factor in increased memory usage.

Can certainly be done for RCM3200.  Can probably be done on RCM2200.

We will have to place a limit on how frequently it checks for events to avoid excessive CPU usage.  The RCM2200 rate will be significantly slower than the RCM3200 rate.

Summary of typical memory usage on DEBROS controllers (without CA server)

Total RAM available:

RCM2200 controllers (84 in service):  	512K

RCM3200 controllers (>180 in service):	768K

Common to all applications (includes DEBROS and Softools libraries):

Root memory (out of possible 56K):

Code:  				  12K

Data:   				  31K

Stack segment:			    4K

Extended memory (for variables, tables, etc):

Code:				217K

Data:				  75K

Extended memory (fixed allocations during startup)

System log and state info:		  17K

Socket buffers:			  60K

Stacks for all processes:		  27K

Far heap:				  32K *

Allocated at run time:

Flash file system buffers:		4-16K (4K/open file, max 4 at present)

* The far heap supports allocating and freeing blocks of memory at run time, which the xalloc() function (used for most of the startup allocations) does not.  Since dynamically allocating and freeing blocks of memory in an RTOS must be avoided for critical functions (because memory may not be available), use of the far heap is limited to non-critical functions like the software “scope” command.

Summary of available resources for existing applications

Application	Processor	Root memory	Extended memory	File space (Flash) *

ECROven	2000	6K		34K			112K

DigPhaseCtlr	3000	2K		240K			104K

DVR	3000	6K		305K			152K

LLRFTurnOn	3000	9K		310K			152K

ModularPS	3000	4K		281K			124K

PS8Rack	3000	4K		295K			140K

TCPS	3000	8K		296K			124K

* The current flash file system code is limited to 12 files.  All applications currently require 6 of the 12 file nodes to operate normally.  The file system is used by the Download Manager, so changing it would require we write our own Download Manager.

Current Limits/settings for various DEBROS resources

Max # of processes:				20

Max # open files on flash system:		  4

Max # of TCP sockets:				12

Max # of UDP sockets:				  3

Max # of open files (all processes):		64

Max # of open files (per process):		16

Max # of different file systems (drivers):	 	  8

Max # remote files open (all processes):	 	  6

Max # FIFOs (all processes):			  8

Max atomic FIFO write:			        1024 bytes

Stack size per task (except kernel):	        1365 bytes (limited to integer division of 4K)